Federal Democratic Republic of Ethiopia

Occupational Standard

Small and Heavy ARMAMENTS SERVICING and maintENANCE

NTQF Level II-III

Introduction

Ministry of Education

June 2016

Introduction

Ethiopia has embarked on a process of reforming its TVET-System. Within the policies and strategies of the Ethiopian Government, technology transformation – by using international standards and international best practices as the basis, and, adopting, adapting and verifying them in the Ethiopian context – is a pivotal element. TVET is given an important role with regard to technology transfer. The new paradigm in the outcome-based TVET system is the orientation at the current and anticipated future demand of the economy and the labor market

The Ethiopian Occupational Standards (EOS) is the core element of the Ethiopian National TVET-Strategy and an important factor within the context of the National TVET-Qualification Framework (NTQF).They are national Ethiopia standards, which define the occupational requirements and expected outcome related to a specific occupation without taking TVET delivery into account.

This document details the mandatory format, sequencing, wording and layout for the Ethiopia Occupational Standard which comprised of Units of Competence.

A Unit of Competence describes a distinct work activity.  It is documented in a standard format that comprises:

- Occupational title, NTQF level

- Unit code

- Unit title

- Unit descriptor

- Elements and Performance criteria

- Variables and Range statement

- Evidence guide

Together all the parts of a Unit of Competence guide the assessor in determining whether the candidate is competent.

The ensuing sections of this EOS document comprise a description of the respective occupation with all the key components of a Unit of Competence -

- chart with an overview of all Units of Competence for the respective level  including the Unit Codes and Unit of Titles

- contents of each Unit of Competence (competence standard)

- occupational map providing the Technical and Vocational Education and Training (TVET) providers with information and important requirements to consider when designing training programs for this standards, and for the individual, a career path

UNIT OF COMPETENCE CHART

Occupational Standard: Small and Heavy Armaments Servicing and Maintenance

Occupational Code: IND SHA

NTQF Level II IND SHA2 02 0616 Weld Using Basic Shielded Metal Arc Welding Process IND SHA2 01 0616 Prepare Basic Engineering Drawing IND SHA2 03 0616 Perform Welding Using Oxyacetylene Process IND SHA2 05 0616 Apply Basic Protective Coating IND SHA3 04 0616 Carry-out Heat Treatment IND SHA2 07 0616 Operate Basic Woodworking Machines IND SHA2 08 0616 Perform Pistols Servicing and Maintenance IND SHA2 11 0616 Perform Repairing of Light Machine Guns IND SHA2 12 0616 Perform Painting Operation IND SHA2 13 0616 Participate in Workplace Communication IND SHA2 14 0616 Work in Team Environment IND SHA2 15 0616 Develop Business Practice IND SHA2 16 0616 Standardize and Sustain 3S

IND SHA2 06 0616 Maintaining and Repairing Electrical/ Electronic Devices IND SHA2 09 0616 Perform Mortars Servicing and Maintenance IND SHA2 10 0616 Perform Repairing of Sub-machine Guns

NTQF Level III IND SHA3 01 0616 Read and Interpret Machine Drawing IND SHA3 02 0616 Set Assembly Stations IND SHA3 03 0616 Perform Precision Assembly IND SHA3 04 0616 Repair of Medium Machine Guns IND SHA3 05 0616 Repair of Cannons IND SHA3 06 0616 Set up Pneumatic and Hydraulic Systems IND SHA3 07 0616 Repair of 122mm D-30 Howitzer IND SHA3 08 0616 Repair of 130mm M-46 Field Gun IND SHA3 09 0616 Repair of Tank Guns IND SHA3 10 0616 Servicing of 155 APU Howitzer Gun IND SHA3 11 0616 Servicing of 152 mm2C19 Howitzer IND SHA3 12 0616 Monitor Implementation of Work Plan/Activities IND SHA3 13 0616 Apply Quality Control IND SHA3 14 0616 Lead Workplace Communication IND SHA3 15 0616 Lead Small Teams IND SHA3 16 0616 Improve Business Practice IND SHA3 17 0616 Prevent and Eliminate MUDA

NTQF Level II

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Prepare Basic Engineering Drawing |
| Unit Code | IND SHA2 01 0616 |
| Unit Descriptor | - This unit covers the knowledge, skills and attitudes required to perform manual drafting to produce simple engineering drawing. |

| Elements | Performance Criteria |
| --- | --- |
| - Prepare or make changes to existing drawings | - Drafting principles are applied to produce a drawing that is consistent with standard operating procedures. - Dimensions, notes and specifications are indicated in the drawing in accordance with drafting principles and standards. - Drafting is accomplished safely and to prescribed procedures. - Completed drawing is presented for approval in accordance with standard operating procedures. |
| - Prepare engineering parts list | - Component parts are identified and organized by component type and/or in accordance with company/customer requirements. - Drawings and/or parts lists records are completed in accordance with standard operating procedures. |
| - Issue approved drawing | - Approved drawing and/or parts lists are copied and issued to relevant personnel in accordance with standard operating procedures. - Approved drawings and/or parts lists are stored and catalogued in accordance with standard operating procedures. |

| Variable | Range |
| --- | --- |
| Drafting principles | - May include but not limited to: - local standards - international standards |
| Records | - May include but not limited to: - cataloguing - issuing security classifications - filing - preparing distribution lists |
| Issued | - May include but not limited to: - hard copy - photographic - slide or transparency form including presentation as a single drawing and/or with other drawings - support documentation as a package |
| Relevant personnel | - May include but not limited to: - supervisor - technical personnel - manufacturers - suppliers - contractors - customers |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | - Demonstrate knowledge and skills to: - identified drawing requirements - prepared engineering .drawing or made changes to existing drawing - prepared engineering parts list - issued approved drawing |
| Underpinning Knowledge and Attitudes | - Demonstrate knowledge of: - Types and uses of drafting equipment and drawing instruments - Requirements and purpose of the engineering drawing and/or parts list - Sources of relevant data/information - Drafting principles to be applied in the preparation of drawing - Drawing symbols and standards - Isometric, orthographic and exploded view drafting - Tolerances and fits - Shop mathematics (geometric principles and trigonometric functions - Types and forms of supply of engineering materials - Types and uses of measuring instruments (scale, steel rule, - Basic machine shop operations - Procedures in checking, recording, copying and issuing completed drawings and/or parts lists - Procedures for safe handling, filing and storage of drawings and/or parts lists - Pattern development procedures for sheet metal work - Procedures in issuing approved drawings and/or parts lists - Safe work practices |
| Underpinning Skills | - Demonstrate skills of: - Using drafting equipment and instruments - Using measuring instruments - Reading and interpreting drawings and sketches - Performing basic mathematical computations - Producing/changing drawing to conform with the relevant standards - Producing the component parts list with part name, description of part, material specification or part number, quantities and all other details specified by the customer and/or organizational procedures - Recording completed drawings and or parts lists in accordance with standard operating procedures - Copying and issuing approved drawings and/or part lists - Communication skills |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit of Title | Weld Using Basic Shielded Metal Arc Welding Process |
| Unit Code | IND SHA2 02 0616 |
| Unit Descriptor | This unit covers preparing materials, selecting and setting up the welding equipment, carrying out basic shielded metal arc welding (SMAW) and inspecting for and correcting defects, in a range of fabrication activities |

| Elements | Performance Criteria |
| --- | --- |
| - Prepare materials and equipment | - Weld requirements are identified from specifications and/or drawings. - Materials are assembled / aligned to specification, where required. - Welding equipment and electrodes are identified. - Welding equipment is assembled and set up. |
| - Minimize and rectify distortion | - Distortion prevention measures are selected appropriate to material and process. - Distortion is minimized by following standards procedures and techniques 2.2 Distortion is rectified. |
| - Weld to job specifications using SMAW | - Weld deposit is made to specification. - Joints are cleaned to specifications - Weld joints are visually inspected for conformance to specifications. - Defects are removed with minimum loss of sound metal using correct and appropriate techniques and tools. |
| - Assess weld quality and rectify faults | - Weld joints are visually inspected against specifications. - Defects are removed using appropriate methods for the given task. - Weld records are correctly completed and maintained. |
| - Maintain weld records as required | - Weld records are correctly completed according to workplace procedures. - Weld records are reported to concerned body according to workplace procedures. - Necessary documentations are accomplished and documents filed/kept in accordance to workplace specifications. |

| Variable | Range |
| --- | --- |
| Welds | May include but not limited to: - Welds would be fillet and butt in all positions |
| Materials | May include but not limited to: - Materials used would include carbon or stainless steel - Preparation of materials including preheating, setting up of jigs, fixtures, clamps, etc., joint preparation e.g. beveling AC or DC welding machines |
| Distortion prevention measures | May include but not limited to: - Remedial action using thermal processes including oxy acetylene and air arc equipment - Grinding devices may also be used |
| Rectified used | May include but not limited to: - preheating, setting up of jigs, fixtures, clamps, etc. |
| Defects | May include but not limited to: - Porosity, slag inclusions, discontinuities, lack penetration, undercut |
| Occupational Health and Safety (OHS) | May include but not limited to: - protective clothing and equipment (include that prescribed under legislation, regulation and workplace policies) - use of tools and equipment, - workplace environment and safety, handling of materials, - use of firefighting equipment, use of first aid equipment, - hazard control and hazardous materials and substances |
| Tools, equipment and materials | May include but not limited to: - Hand and power tools, - Measuring equipment, - Relevant welding machines - Relevant welding electrode and materials - Gloves, shields and other protective (cloths and leather shoes) equipment - AC or DC welding machines and their accessories |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | - Demonstrate knowledge and skills to: - Prepared materials for basic shielded metal arc welding(SMAW) - Assembled and setting up welding equipment - Welded to job specification using basic SMAW - Minimized and rectified distortion - Identified the properties and characteristics of a wide range of materials - Applied weld procedures, techniques and standards - Identified different welder systems such as numbering, bar coding, paint coding, letter stamps - Applied safety requirements and safe welding practices - Used personal protective equipment for SMAW |
| Underpinning Knowledge and Attitudes | Demonstrate knowledge of: - In-depth knowledge of the properties and characteristics of a wide range of materials - Different welder identification systems such as numbering, bar coding, paint coding, letter stamps - Safety requirements |
| Underpinning Skills | Demonstrate skills to - Weld procedures and requirements - Different welder identification systems such as numbering, bar coding, paint coding, letter stamps - Safe welding practices - use and application of personal protective equipment for SMAW |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Perform Welding Using Oxyacetylene Process |
| Unit Code | IND SHA2 03 0616 |
| Unit Descriptor | This unit covers competence in performing oxyacetylene welding, carried out using a range of materials for general fabrication. It focuses on oxyacetylene welding process in fabrication and assembly of metals. |

| Elements | Performance criteria |
| --- | --- |
| - Prepare welding equipment and materials | - Correct welding equipment is selected. - Correct welding consumables are selected. - Welding equipment, including cylinders, regulators, hoses, torches and tips are assembled and set up safely in accordance with standard operating procedures. - Preparing materials are to achieve required weld specification. |
| - Weld joints | - Welding equipment is adjusted correctly and safely. - Materials are welded to standard in all positions. - Instructions, symbols, specifications are interpreted correctly including bead size, bead placement, reinforcement etc. and in accordance with weld procedure sheet, if available, and standard operating procedures. - Welding procedure and specifications (WPS) and Occupational Health and Safety (OHS) - procedures/measures are followed/ observed throughout the welding operation. |
| - Correct faults | - Welding joints are visually inspected against specifications. - Weld defects are identified. - Defects are removed with minimum loss of sound metal using correct and appropriate techniques and tools. |
| - Maintain weld records | - Weld records are filled up in accordance with specifications and standard operating procedures. - Weld records are maintained in accordance with specifications and standard operating procedures. - Weld records are filed / stored in accordance with enterprise system and standards. |

| Variable | Range |
| --- | --- |
| Consumables | - May include but not limited to: - Fuel gases including oxyacetylene, oxyacetylene generator, LPG, hydrogen etc., cylinders, regulators, hoses, torches, tips, range of filler rods and fluxes |
| Preparing materials | - May include but not limited to: - Preheating, setting up of jigs, fixtures, clamps, etc., joint preparation e.g. beveling |
| Equipment | - May include but not limited to: - hand and power tools, - measuring equipment, - guillotines, soldering equipment, - oxyacetylene and accessories |
| Materials | - May include but not limited to: - Low carbon steel, plate, pipe, tube and round bar |
| Weld | - May include but not limited to: - Fillet and butt in flat, horizontal, vertical and over head positions |
| Occupational Health and Safety (OHS) | - May include but not limited to: - protective clothing and equipment, - use of tools and equipment, - workplace environment and safety, handling of materials, - use of firefighting equipment, use of first aid equipment, - hazard control and hazardous materials and substances personal protective equipment is to include that prescribed under legislation, regulation and workplace policies and practices |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | - Demonstrate knowledge and skills to: - performed preparatory requirements - explained purpose and examples of pre-welding and post welding; purpose of reinforcing areas to be welded; fuel gas properties and applications - performed heating of the weld materials - determined appropriate settings for the given task and the selected equipment/consumables - performed methods of weld defect removal and their application - identified material and consumable properties and characteristics - performed post treatments - observed recording procedures - observed safe welding practices - used personal protective equipment for oxy acetylene welding - observed relevant hazards and control measures related to the competence |
| Underpinning Knowledge and Attitudes | - Demonstrate knowledge of: - preparatory requirements - purpose and examples of pre-welding and post welding - appropriate settings for the given task and the - selected equipment/consumables - purpose of reinforcing areas to be welded - material and consumable properties and characteristics - fuel gas properties and applications - post treatments - recording procedures - relevant hazards and control measures related to the unit |
| Underpinning Skills | - Demonstrate skills of: - apply operational safety requirements - access, interpret and apply technical information - apply hand-eye coordination - read sketches or basic drawings - identify and select from a range of welding equipment and accessories - identify and match equipment with specified tasks - heating of weld materials - comply with environmental requirements - methods of weld defect removal and their application - safe welding practices and use / application of personal protective equipment for oxyacetylene welding |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Carry-out Heat Treatment |
| Unit Code | IND SHA3 04 0616 |
| Unit Descriptor | This unit covers the necessary knowledge, attitude and skills in carrying out simple heat treatment of materials. This unit focuses on performing straightforward heating/quenching, tempering and annealing of ferrous and non-ferrous metals. Work would normally cover one-off processes or processes undertaken as incidental to trade work. |

| Elements | Performance Criteria |
| --- | --- |
| - Prepare for work | - Job requirements are determined from engineering drawing, job sheet or verbal instructions - Appropriate heating process and/or procedure is selected for the given job - Equipment is set up according to standard operating procedures and manufacturers’ instructions - Work area is made ready and safe for work |
| - Operate heating equipment | - All safety procedures are observed - Appropriate heating equipment operating procedures are followed - Appropriate equipment adjustments are made. - Material is treated to achieve required result. |
| - Clean up | - Work area is cleared and materials disposed of or recycled in accordance with legislation and workplace procedures - Tools and equipment are cleaned, checked, maintained and stored in accordance with manufacturers’ recommendations and workplace procedures - Documentation is completed in accordance with workplace requirements |

| Variable | Range |
| --- | --- |
| Heating process | - May include but not limited to: Heating/quenching, tempering and annealing |
| Heating equipment | - May include but not limited to: Furnaces |
| Material | Ferrous and non-ferrous metals of various types and thicknesses |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate: - performed basic incidental heat/quenching, tempering and annealing of ferrous and non-ferrous metals by a range of methods, which may include oxy acetylene, LPG gas equipment, forge, and use to heat/quench temper and anneal materials to specifications |
| Underpinning knowledge and Attitudes | Demonstrates knowledge of: - characteristics and applications of heating/quenching, tempering, annealing processes - specifications for heating, quenching, tempering, and annealing - process for heating/quenching, tempering, annealing, different materials - operating/adjusting heating equipment - hazards and control measures associated with heating/quenching, tempering, annealing, including housekeeping - use and application of personal protective equipment - safe work practices and procedures |
| Underpinning skills | Demonstrates skills of: - reading, interpreting and following information on written job instructions, specifications, standard operating procedures, manufacturer instructions, charts/lists and drawings - checking and clarifying task-related information - following verbal instructions - orally reporting routine information - selecting appropriate processes for heating/quenching, tempering, annealing - setting up, adjusting and operating equipment |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Apply Basic Protective Coating |
| Unit code | IND SHA2 05 0616 |
| Unit Descriptor | This unit covers knowledge, attitudes and skills needed in spraying pre-treatments and protective coatings not limited to brush, roller and conventional spray equipment at a basic level. |

| Elements | Performance Criteria |
| --- | --- |
| - Determine job requirements | - Work requirements are determined from job sheet, instructions, drawings or visual inspection. - Required protective coating materials are identified according to job specification - Required protective coating application equipment is identified according to job requirements. - Work site is prepared for application of protective coating - Appropriate personal protective equipment for coating application is selected and used properly in accordance with manufacturers’ specifications and standard operating procedures. - Ancillary support attachments are identified and used. |
| - Prepare workpiece for coating | - Surface condition is inspected for readiness for application of protective coating according to specification. - Unsuitable work pieces/surfaces and fabrication defects are identified and appropriate remedial action or reporting is undertaken in accordance with standard operating procedures. - Components are masked where protective coating application is not specified. - Conditions for overspray are identified |
| - Prepare equipment | - Required plant and equipment basic operations are understood. - Routine maintenance is undertaken on plant and equipment in accordance with standard operating procedures - Status/reports are recorded by proforma or orally in accordance with standard operating procedures. - Conventional coating application equipment is assembled in accordance with equipment requirements and standard operating procedures. |
| - Apply single pack coating | - Coating product type, solvent, uses, mixing procedure, clean-up and safety requirements are identified as appropriate. - Correct method of determining wet film thickness in accordance with specified dry film is demonstrated. - Coating material is thinned to suit the application method and to achieve required film thickness. - Coating is applied using specified application method and standard operating procedures. - Coating schedules can be outlined for metal and non-metal materials. - Coating application and curing technique are monitored according to standard operating procedures. |
| - Inspect finish surface | - Surface finish is assessed for profile size differences and uses. - Coating thickness is determined using appropriate instruments and results are compared with job specifications. - Total surface is inspected for conformance to specification in accordance with standard operating procedures. - Inspection results are recorded and reported in accordance with standard operating procedures. |
| - Clean and store equipment | - Conventional coating application equipment is cleaned, disassembled and inspected for damage. - Faulty equipment is recorded and reported to appropriate personnel in accordance with standard operating procedures. - Coating application equipment is stored in accordance with standard operating procedures. - Personal protective equipment is maintained in accordance with manufacturers’ specification and standard operating procedures. |

| Variable | Range |
| --- | --- |
| Prepared | - May include but not limited to: - Visual inspection for contamination, masking out, racking, identification of correct materials to be used, ensuring cleanliness of work area and equipment, operational testing of equipment |
| Personal protective equipment | - May include but not limited to: - Includes hand protection, full body protection, respirators, air fed hoods and foot protection. Noise and heat protection may also be necessary |
| Coating operation | - May include but not limited to: - Spray application, roller, dip coating, etc. |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate: - Applied basic protective coatings |
| Underpinning knowledge and Attitudes | Demonstrates knowledge of: - work to be undertaken and specifications applying to the work - correct interpretation of all relevant information - safety issues, adequate precautions, awareness of other site factors that could be affected by the work - deviation from specified surface finish/condition - standard workplace procedures for identifying unsuitable work items - method of locating areas to be protected from coating process and masked - the areas subject to overspray and requiring protection - operation of plant and equipment using standard operating procedures - standard operating procedures for plant and equipment maintenance - the requirements for completion and processing of maintenance reports - assembly specifications and procedures - workplace procedures for identifying coating types and processing solvents, mixing and safe handling practices - the workplace procedures for determining the wet film thickness of a coating from the specified dry film thickness - calculations using a specified formulation - standard operating procedure for thinning coating materials for use in applying the specified film thickness coating to a substrate - standard operating procedures to apply protective coatings to comply with an established standard - procedures for controlling coating application and curing techniques - standard operating procedure for disassembly, cleaning and checking - standard operating procedures for recording and reporting defective parts - standard operating procedures for storage and protection of equipment - standard operating procedures and other relevant standards for assessing the profile of the surface finish - dry film thickness testing instruments - standard operating procedures for surface inspection, including the extent and detail of inspection as required - requirements for completion and processing of inspection reports - standard procedure for identifying and selecting the required personal protective equipment - the workplace standard procedure for the use of personal protective equipment - the use and selection of appropriate supports to use with personal protective equipment in standard operational practices - standard procedures and manufacturers’ specifications for inspecting and maintaining personal protective equipment in the workplace |
| Underpinning Skills | Demonstrates skills of: - obtaining all relevant drawings, job sheets, specifications and instructions in accordance with workplace procedures considering all relevant information and job requirements - preparing site with due regard to OHS requirements including site safety, clear working space, other materials/structures/personnel in the vicinity, isolation of work site where required - undertaking comprehensive inspection and considering specifications during inspection - using standard workplace procedures to identify, select and apply the appropriate treatment or actions to rectify items with surface or fabrication defects - identifying surface required as 'no paint areas' and protecting these using standard masking procedures and techniques - undertaking precautions to prevent overspray in the workplace using standard procedures - undertaking routine maintenance on plant and equipment in accordance with standard operating procedures - preparing all required maintenance records/reports and details communicated - assembling equipment in accordance with manufacturers’ specifications and standard operating procedures - identifying coating type and appropriate solvents and standard workplace procedures required for mixing processes, clean-up and safe handling - using workplace procedures for determining the wet film thicknesses of a coating from the specified dry film thickness - calculating required thickness in accordance with product volume solids - applying standard operating procedures for thinning coating materials and applying the specified film thickness coating to a substrate - applying protective coating to comply with an established standard using specified methods and standard operating procedures - controlling coating application and curing techniques using standard operating procedures - undertaking disassembly, cleaning and checking for functionality of spraying equipment and associated items in accordance with standard operating procedures - using standard operating procedures to report on any damage or faulty parts and communicating with appropriate personnel - following procedure for storage including any hazard reduction and/or protection of equipment and components - checking surface condition of the work piece, including profile size properties and problems according to standard operating procedures and other acceptable standards - determining thickness using mechanical, electronic or other appropriate instruments and testing results compared with job specifications, drawings etc. - undertaking comprehensive inspection as required by standard operating procedures - preparing all required inspection records/reports and details communicated - selecting and using appropriate personal protective equipment in workplace operations in accordance with standard procedures - selecting the appropriate supports to use with personal protective equipment in standard operational practices - checking personal protective equipment items for serviceability in accordance with manufacturers’ specifications - recording and reporting faulty items to appropriate personnel using standard workplace procedures |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Maintaining and Repairing Electrical/ Electronic Devices |
| Unit code | IND SHA2 06 0616 |
| Unit Descriptor | This unit is designed to equip trainees with basic electrical knowledge about source of electricity, electrical hand tools, splicing and joining, termination, soldering, electrical quantities, measuring instruments AC and Dc circuits, electrical devices, electrical components, soldering and de-soldering techniques on PCB, electronic circuits , Digital and analog system, Binary numbers, Digital Codes, Logic gates, logic families, Sequential logic circuits, Memory types and programmable logics, Mass storage, Interfacing circuits and microprocessor. It contains both the theoretical part and practical activities the trainee should pass through to acquire the basic competence (skill, knowledge and attitude) in accomplishing organizational activities relating to its coverage. |

| Elements | Performance criteria |
| --- | --- |
| 1. Prepare for work | - Work instructions are used to determine job requirements, including method, processes and equipment. - Job specifications are read and interpreted - OHS requirements, including personal safety needs, are observed throughout the work. - Equipment and tooling are identified and checked for safe and effective operation. - Main power supply is disconnected - Procedures are determined to minimize task time |
| 2. Identify Source of electricity | - Source of electricity is identified - production of each source of electricity is elaborated using diagrams - Common types of electrical hand tools are selected properly |
| 3. Perform Splicing, joining and Termination | - Splicing and termination of conductors are defined clearly. - Advantage of splicing and termination are mentioned correctly - Methods of splicing and termination are described - Types of splicing and termination are mentioned correctly. |
| 4. Perform Soldering operation | - Soldering procedure is explained. - Soldering materials are listed. - Spliced and joined wires are soldered according to the procedures correctly. - Components are soldered on the PCB following the procedure. - Components are de-soldered from the PCB following the procedure. |
| 5. Identify and Measure Electrical quantities | - Voltage (Electromotive force) is identified and measured using voltmeter according to the operating manual - Current is identified and measured using ammeter according to the operating manual - Resistance is identified and measured using ammeter according to the operating manual - Capacitor is identified and measured using capacitor meter according to the operating manual - Inductor is identified and measured using Inductor meter according to the operating manual - Power is identified and measured using watt meter according to the operating manual - Energy is identified and measured using energy meter according to the operating manual |
| - Analyze AC and DC Circuits | - Parts of DC circuits are explained correctly. - Types of DC circuits are listed - Parameters of AC circuits are explained correctly. - Types AC circuits are mentioned according to its parameters. - Types of load and ohm's law are explained |
| - Identify Basic Electrical Device | - Types of switches are identified - Function of Switches are explained - Types and uses of Fuses are explained. - Types and uses of Circuit breakers are explained. |
| - Identify Electronic Components | - Passive components are identified - Uses or functions of passive components are mentioned - Passive components are tested for their proper functioning according to the specification - Active components are identified - Types of Active components are mentioned. - Uses or functions of active components are mentioned - Active components are tested for their proper functioning according to the specification - Optoelectronic devices are identified. - Integrated circuits are identified. - Types of Integrated circuits are differentiated. |
| - Test and repair Electronic circuits | - Power supply circuits are explained - Parts and types of power supply circuits are mentioned - Operation principles of voltage multiplier circuits are explained - Troubleshooting power supply circuits - Amplifier circuits are explained - Types of amplifier circuits are mentioned - Troubleshooting amplifier circuits. - Wave generating circuits are explained. - Types of Wave generating circuits are mentioned. - Troubleshooting of wave generating circuits is performed. - Operational amplifier (active) Filters are explained. - Types of active filters are mentioned - Troubleshooting of active filters |
| - Familiarize Digital and analog system | - Digital and analog quantities are explained and differentiated. - Logic level and pulse wave forms are explained. |
| - Familiarize Binary numbers | - Binary number systems are explained - Conversion of Binary number systems are explained |
| - Familiarize Digital Codes | - Digital codes are explained - Conversion of Binary numbers to BCD, GRAY, Exess-3 and ASCII are mentioned. |
| - Identify Logic gates | - Symbols and operations of Logic gates are explained. - Types of Logic gates are mentioned. |
| - Interpret circuit logic families | - Interpreting circuit logic families ( TTL, CMOS, NMOS &PMOS) are explained - Application area of logic families is identified. - Troubleshooting of logic gates families. |
| - Operate & apply Combinational logic circuits | - Operation & application of combinational logic circuits are explained. - Different types of combinational logic circuits are mentioned. |
| - Operate & apply Sequential logic circuits | - Operation & application of sequential logic circuits are explained - Different types of sequential logic circuits are mentioned. |
| - Identify Memory types & programmable logics | - Memory types and programmable logics are explained. - Different types of memories are mentioned. |
| - Familiarize Mass storage | - Mass storage devices (Magnetic memories (HD, Tape streamers) are explained - Optical memories (CD, DVD) and Programmable logic devices are explained. |
| - Interfacing circuits | - Interfacing circuits are explained. - Digital to analog (D/A) conversion and Analog to digital (A/D) conversion are explained. |
| - Introduce microprocessor | - Introducing microprocessor - Introducing CPU, - Introducing Memory - Introducing Input/output - Introducing Microprocessor programming - Introducing Microprocessor role in modern control systems |
| - Report the measured results | - Reporting format is prepared - Information is collected and analyzed - Information is organized and recorded - Report is submitted to higher body - Copy of report is filed |

| Variable | Range |
| --- | --- |
| Unit scope | May include but not limited to: - Manual read & understood - Check for appropriate safety devices - Use proper measuring instruments - Identify Source of electricity - Perform splicing and termination - Perform Soldering operation - Identify Measuring instruments - Identify and Measure Electrical quantities - Analyze AC and DC Circuits - Identify Basic Electrical Device - Identify Electronic Components - Testing and repairing Electronic circuits - Differentiate Digital and analog system - Use Binary numbers - Identify Digital Codes - Identify Logic gates - Interpret circuit logic families - operate & apply Combinational logic circuits - operate & apply Sequential logic circuits - Identify Memory types and programmable logics - Identify Mass storage Interfacing circuits & microprocessor - Report the activities performed and results of quantities measured. - Safe handling of manual, safety devices, Proper clothing, Safe handling externals materials & tools, Safe working with electrical power, safe handling of measuring instruments, equipments and devices, Proper handling of report. |
| Occupational Health & Safety (OHS) | May include but not limited to: - Fire prevention and protection - Fire safety and fire extinguisher training |
| Tools and Equipment | May include but not limited to: - Manual, Stationary materials, Hand tool box, Apron, Gloves, wires, connectors, soldering irons, solders, PCBs, switches, fuses, circuit breakers, active and passive components, Electrical circuits, Logic gates, Ammeters ,voltmeters, Watt meters, Energy meters, Frequency meters, Power factor meters, Syncroscope meters, - Local company regulations, Data book and Prepared format. |
| Types and Sources of Information | May include but not limited to: - Instrument’s operation manual, system operation manual Schematic diagram, profile of reports and other relevant specifications |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | - Demonstrate by applying: - Identified and mentioned clearly Source of electricity - Performed different types of splicing and termination - Performed Soldering operation following soldering procedure. - Identified Measuring instruments - Identified and Measured Electrical quantities with Measuring instruments - Analyzed AC and DC Circuits - Identified Basic Electrical Device and their use is explained correctly. - Identified active and passive Components and explained area of application. - Tested and repaired Electronic circuits according to the servicing and operating manuals. - Differentiated Digital and analog system - Understood uses of Binary numbers - Identified different Digital Codes - Identified Logic gates - Interpreted the logic families and their property - Explained the operation & application of Combinational logic circuits - Explained the operation & application of Sequential logic circuits - Identified Memory types and programmable logics - Identified Mass storage, Interfacing circuits & microprocessor - Reported result of work |
| Underpinning Knowledge and Attitudes | - Demonstrates knowledge of: - Safety rules, - Technical drawing - reading and interpreting electrical component symbols and electrical drawings - Basic principle of Measuring Instruments, - fire safety, - Rules of first aid, |
| Underpinning Skills | - Demonstrates skills to: - checking the drawing against job requirements/related equipment in accordance with standard operating procedures - Measuring current, voltage, resistance etc., - Test and repair Electronic circuits |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Operate Basic Woodworking Machines |
| Unit Code | IND SHA2 07 0616 |
| Unit Descriptor | This unit covers the knowledge, skills and attitudes required to operate basic woodworking machines used for sawing, planning, sanding and drilling operations for the production of furniture. |

| Elements | Performance Criteria |
| --- | --- |
| 1. Prepare for work | 1.1 Work instructions are used to identify job requirements, including quality, materials, equipment, and quantities 1.2 Workplace health and safety requirements, including ventilation requirements and personal protection needs, are observed throughout the work 1.3 Material for machining is selected and inspected for quality 1.4 Tools, equipment and machines, cutting tools and jigs are identified and checked for safe and effective operation |
| 2. Operate machines | 2.1 Safety equipment, including emergency stops, gauges, guards and controls are checked. 2.2 Machine start-up procedure is carried out in accordance with manufacturers’ instructions. 2.3 Material is fed into machine in accordance with manufacturers’ instructions, safe handling procedures and standard workplace operating procedures. 2.4 Machine is operated in accordance with its designed capacity and purpose, tooling requirements and to manufacturers’ recommendations. 2.5 Machine operation is monitored to ensure product quality and output. 2.6 Waste quantities are checked and minimized. 2.7 Items that do not meet quality requirements are recycled or discarded according to workplace procedures. 2.8 Defective machines are identified and reported to appropriate persons |
| 3. Clean up work area and maintain equipment | 3.1 Material that can be reused is collected and stored following storage procedure. 3.2 Waste and scrap are removed following OHS procedures. 3.3 Equipment and work area are cleaned and inspected for serviceable condition in accordance with workplace procedures 3.4 Replacing and adjusting minor problems on hand tools and portable machines is performed according to guidelines. 3.5 Unserviceable/un adjustable equipment is reported in accordance with workplace procedures |

| Variable | Range |
| --- | --- |
| Workplace health and safety requirements | May include but not limited to: - OHS requirements include legislation, material safety management systems, hazardous substances and dangerous goods code and local safe operating procedures - Work is carried out in accordance with legislative obligations, environmental legislation, relevant health regulations, manual handling procedures and organization insurance requirements |
| Material | May include but not limited to: - Material to be machined may include but is not limited to: solid timber and manufactured board |
| Tools and Equipment | May include but not limited to: - Circular saw , surface planner, jointer/thicknesser planer, belt sander, disk sander, drill machine and band saw |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Demonstrates Knowledge and skills of: - Identify work requirements - Apply safe handling requirements for equipment, products and materials, including use of personal protective equipment - Identify materials used in the work process - Follow work instructions, operating procedures and inspection processes to: - minimize the risk of injury to self or others - prevent damage to goods, equipment and products - maintain required production output and product quality - Operate and monitor the machines and complete the tasks identified in the Range Statement for: - one saw - one planer - one drill - one sander - Work effectively with others |
| Underpinning Knowledge and Attitudes | Demonstrates Knowledge of: - The characteristics, uses and limitations of basic woodworking machines - General workplace quality standards and procedures - Workplace safety policies and procedures - Procedures for reporting machinery faults and material defects - Mathematical ideas and techniques. - Basic arithmetical computing - Checking techniques to identify, avoid or solve work problems - Using the workplace technology related to machine operations, including tools, equipment and basic measuring devices. |
| Underpinning Skills | Demonstrates skills of: - operating basic woodworking machines - maintaining basic woodworking machines - performing housekeeping |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Perform Pistols Servicing and Maintenance |
| Unit Code | IND SHA2 08 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding servicing, maintenance as well as repairing different mechanisms of various pistol variants (.45, makarov, revolver, .35 etc. pistols). |

| Elements | Performance Criteria |
| --- | --- |
| Maintain Slide Mechanism | 1.1 Disassembled and check ,inspect firing pin extrusion as per specifications 1.2 Remove dents, nicks scratches from firing pin housing and replace damaged firing pin and firing spring 1.3 Check and inspect extractor proper functioning 1.4 Remove dents, nicks, and scratches from extractor and the housing. 1.5. Damaged pins springs are replaced 1.6.Broken safety locks are replaced 1.7. Assembled and check malfunction of the slide mechanism. |
| 2. Maintain Recoil Mechanism | - Disassemble recoil mechanism and correct information is accessed and interpreted from Manufacturer’s and designer’s specifications. - Springs are manufactured using approved method based on workplace guidelines. - Quality of springs is checked and tested according to the standard working procedures. - Broken or weak springs are replaced correctly on its right position according to manufacturer’s specifications. - Dents, scratches, bulges on spring guide (barrel) are removed. - Assembled and check malfunction of the recoil mechanism |
| 3. Maintain Receiver Mechanism | 3.1.Disassemble receiver mechanism and dents, scratches, bulges on hammer are removed by filing using an appropriate file, badly damaged hammer is replaced. 3.2. Bend sears are adjusted, damaged sears are replaced. 3.3. Dents, scratches, bulges on slide stops are removed using an appropriate. 3.4. Damaged cooking lever leafs are rejected. 3.5. Assembled and check malfunction of the receiver mechanism |
| 4. Maintain Magazine Mechanism | 4.1 Disassemble magazine and Specific problems of magazine are identified following standard work procedures. 4.2 Bend followers and plates are adjusted. 4.3 Badly damaged plates followers are replaced to make more functional 4.4. Quality of springs is checked and tested according to the standard working procedures. 4.5. Broken or weak springs are replaced correctly on its right position according to manufacturer’s specifications 4.6. Assembled and check malfunction of the magazine mechanism |

| Variable | # Range |
| --- | --- |
| Damage | May include but not limited to: # Broken firing pins # weak firing pin spring # weak magazine spring |
| Quality | May include but not limited to: # Strength (elasticity) of the springs |
| Occupational Health & Safety (OHS) | May include but not limited to: # OHS requirements are to in accordance with legislations/regulations enterprise safety policy and procedure including protective closing, equipment tool and hazard control. # Follow Occupational health and safety procedures designated for the task # Check and fulfill required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: # SPTA # Hammer, saw, wire brush, sand paper, scraper, wilding equipment and material, file, different machine, and special tools .45 pistol, makarov, revolver, .35 pistol. |
| Types and Sources of Information | May include but not limited to: # Information source may include variable written and graphical instructions, work bulletins, data sheet, diagrams or sketches # Safety work procedures/manual # Workplace guidelines # Manufacturer’s diagrams, charts # Manufacturer’s catalogue/specification manual. # Manufacturer’s service and operation manuals # Design specification manual # Repair request documentation # Manufacturing and designing specifications and instructions # Records and reports # Internet # Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate: # Descried slide, recoil receiver and magazine mechanism maintenance procedures # Demonstrated safe work practices/procedures # Described basic designing and manufacturing procedures # Manufactured springs, tested quality and replaced worn out. # Identified specific problems of lock, hammer, sear, cooking lever, firing pin, recoil spring guide ,plate and follower of magazine, # Re-machined deposit weld material is re-machined for broken or bent belt according # Documented performances and reported to appropriate personnel |
| underpinning knowledge and attitude | Demonstrates knowledge of: # Fundamentals small arms # Fundamentals of open sight principles # Knowledge of interaction of pistol parts # Operating characteristics of pistols |
| Underpinning skill | Demonstrates skills of: # Disassembling pistols # Troubleshooting pistols # Assembling the pistol # Checking the functionality by firing |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: # Interview / Written Test # Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Perform Mortars Servicing and Maintenance |
| Unit Code | IND SHA2 09 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding replacement, maintenance as well as repairing different parts of mortar variants (60mm, 82mm, 120mm etc mortars). |

| Elements | Performance Criteria |
| --- | --- |
| - Repair barrel complete with base cap | 1.1 Disassemble barrel and Inspect the degree of nicks. 1.2. Slight nicks on barrel are removed by removing minimum layer of material 1.3. Rusts and powder gases disposition is removed from the bore using an appropriate material and Use chemicals for farther cleaning 1.4. Adjust and measure the extrusion of firing pin according to the specific manual. 1.5. Cracked and badly damaged barrel is rejected 1.6 Assembled and check malfunction of the barrel. |
| - Repair base plate | 2.1. Disassembled base plate and identify the problem. 2.2. Cracks and minor damaged on base plates are welded. 2.3. Badly damaged base plate is replaced 2.4. Assembled and check malfunction of the base plate. |
| - Repair Bipod with Coarse Cross-Level Mechanism | 3.1. Disassembled Bipod with Coarse Cross-Level Mechanism and identify the problem. 3.2.Adjust dents, scratches on threads on the mechanism 3.3.Replace broken chains 3.4 Assembled and check malfunction of the Bipod with Coarse Cross-Level Mechanism |
| - Repair laying mechanism | 4.1.Disassembled laying mechanism and identify the problem 4.2. Gears are inspected 4.3 Gear teeth are reconditioned by machining and frilling 4.4 Gears with worn-out teeth are replaced 4.5 Key-ways are reconditioned 4.6 Gears are clean, lubricated with oil 4.7 Assembled and check malfunction of laying Mechanism |
| - Repair Yoke With Shock absorber | 5.1. Disassembled laying mechanism and identify the problem 5.2. Springs are inspected and damaged springs are replaced according to specifications. 5.3. Rust and dents are removed from outer and inner cylinders using an appropriate material. 5.4 Assembled and check malfunction of Yoke With Shock absorber. |
| - Perform bore sighting | 6.1 perform leveling the gun 6.2 perform assigning target point 6.3 perform aligning target point with strap line |

| Variable | Range |
| --- | --- |
| Occupational Health & Safety (OHS) | May include but not limited to: - OHS requirements are to in accordance with legislations/regulations enterprise safety policy and procedure including protective closing, equipment tool and hazard control. - Follow Occupational health and safety procedures designated for the task - Check and fulfill required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: - SPTA - Hammer, saw, wire brush, sand paper, scraper, wilding equipment and material, file, different machine, Crain and special tools, 60mm mortar, 82mm mortar |
| Types and Sources of Information | May include but not limited to: - Information source may include variable written and graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate: - Describe barrel maintenance procedures - Demonstrated safe work practices/procedure - Testing springs, quality and replaced worn out. - Identified specific problems of base plate, bipod, laying mechanism, and shock absorber. - Re-machined deposit weld material on broken and cracked parts. - Documented performances and reported to appropriate personnel |
| Underpinning knowledge and attitude | Demonstrates Knowledge of: - Tactical use of mortars - Working principle of mortars - Stabilization principles of smooth bore barrels |
| Underpinning skill | Demonstrates Skills to: - Dismantling all parts of mortar gun - Check the functionality of all parts (laying mechanism and yoke) - Manufacture parts which are damaged - Replace malfunctioning parts (laying mechanism) - Complete assembling and checking the mortar - Perform bore sighting |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Perform Repairing of Sub-machine Guns |
| Unit Code | IND SHA2 10 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding servicing, maintenance as well repairing different parts and mechanisms of sub machine guns (AK-47, Sniper etc). |

| Elements | Performance Criteria |
| --- | --- |
| 1 Repair Firing Mechanism | 1.1 Disassemble parts of the firing mechanism and inspect and identify its problem 1.2 Check and inspect the defects of the breech mechanism. 1.3 File worn-out striker assembly. 1.4 Removed allowable dents, nicks, scratches by filing. 1.5 Replace damaged striker 1.6 Replaces damaged firing pin and firing spring. 1.7 Remove dents, nicks scratches from firing pin housing 1.8. Remove dents, nicks, and scratches from extractor and the housing. 1.9. Replace damaged extractor and spring. 2.0. Replace worn out pins, cocking cam, breech block, shaped recess and lugs. |
| 2. Repair Stock and hand guard assembly | 2.1. Disassemble, Check and inspect the problem of the stock and hand guard assembly. 2.2. Cracked and damaged stock and hand guard assembly is maintained using the repair manual specifications. 2.3. Badly damaged stock and hand guard assembly is repaired |
| 3. Repair Operating rod with piston | 3.1. Disassemble, Check and inspect the problem that occurred on the rod and piston. 3.2. Dents, scratches, on the piston head are removed slightly using an appropriate material. 3.3. Smoke, deposition from the piston rod and cylinder are removed. 3.4. Badly bend and damaged rod are rejected for farther maintenance. 3.5. Assembled and check its malfunction. |
| 4.Maintain Bolt assembly | 4.1. Disassemble, Check and inspect the problem that occurred on the bolt assembly 4.2.Disassembled and check ,inspect firing pin extrusion as per specifications 4.3 Remove dents, nicks scratches from firing pin housing and replace damaged firing pin and firing spring 4.4 Check and inspect extractor proper functioning 4.5 Remove dents, nicks, and scratches from extractor and the housing. 4.6. Damaged pins springs are replaced. 4.7. Assembled and check malfunction of the bolt assembly. |
| 5. Repair Barrel and Receiver group | 5.1 Disassemble barrel and identify its problems 5.2 Check and inspect the bore visually 5.3. Measure width and depth using inappropriate tool. 5.4. Damaged lands/rifles beyond the limit are rejected 5.5 Inspect the degree of nicks. 5.6. Slight nicks on barrel are removed by removing minimum layer of material 5.7. Rusts and powder gases disposition is removed from the bore using an appropriate material. 5.8 Use chemicals for farther cleaning. 5.9. Cracked barrel is rejected 5.10 Bend sears are adjusted, damaged sears are replaced. 5.11. Dents, scratches, bulges on slide stops are removed using an appropriate. 5.12.Assemble the barrel and receiver and check its functionality |
| 6.Maintain Rifle bipod | 6.1. Disassemble the bipod and check and inspect. 6.2. Slight bends are adjusted, using an appropriate tool 6.3. Broken bipod is welded. 6.4. Damage springs are replaced. 6.5. After maintenance assemble the bipod and check its functionality |
| 7.Maintain the magazine assembly | 7.1. Disassemble the magazine from the receiver and check and inspect 7.2. Bended magazine catch cover are adjusted using an appropriate tool. 7.3. Badly damaged magazine is rejected. 7.4. After maintenance assemble the magazine and check its functionality |

| Variable | # Range |
| --- | --- |
| Firing mechanism | May include but not limited to: - AK-47 Firing mechanisms - sniper gun Firing mechanisms |
| Bolt assembly | May include but not limited to: - AK-47 bolt assembly - sniper gun bolt assembly |
| Barrel | May include but not limited to: - AK-47 barrel - sniper gun barrel |
| Occupational Health & Safety (OHS) | May include but not limited to: - Using of relevant Uniforms and protective when necessary - Using Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones , gloves etc, - Maintaining fire extinguishers and protection tools at appropriate place - Following Occupational health and safety procedures designated for the task - Checking and fulfilling required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: - SPTA - Welding machines - Belching chemicals(anti rust) - Manufacturing machines - Hammer, mallets, wire brush etc - Measuring tools - Wire brush, sand paper - Wool and cloth |
| Types and Sources of Information | May include but not limited to: - Information source may include variable written and graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate: - Described Maintain procedures and precautions of barrel with receiver assembly - Followed work instructions and procedures to determine job requirements - Identify the problems on the firing mechanism - Describe the maintenance procedure of bolt assembly, stock and hand guard assembly. - Maintained , piston rod and (cylinder) - Identified and taken measure for gas leakages - demonstrated safe work procedures - Identified proper working tools - Cleaned up work area and maintained equipment |
| Underpinning knowledge and attitude | Demonstrates Knowledge of: - Basic manufacturing - Basic designing and specification - General knowledge of maintenance - Work ethics - Basic Metallurgy - Technical drawing - Fundamentals small arms - Fundamentals of optics - Basic mathematics knowledge - Basic writing and speaking knowledge - General knowledge of material management and shop - Basic sheet metal work - Tactical characteristics of sub machine guns - Ballistics characteristics of the guns - Types of automatic operations and their working principle - Interactions of the different parts of the weapons - Usual troubles that encountered on these guns and their remedies |
| Underpinning skill | Demonstrate Skills to: - Complete stripping of the guns - Checking the functionality of springs. - Manufacturing some damaged parts like firing pin. - Cleaning and assembling the weapons. - Checking by firing and adjusting the fore and back sight |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Perform Repairing of Light Machine Guns |
| Unit Code | IND SHA2 11 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding servicing, maintenance as well repairing different parts and mechanisms of light machine guns (PKM and PKT). |

| Elements | Performance Criteria |
| --- | --- |
| 1 Repair Firing Mechanism | 1.1 Disassemble parts of the firing mechanism and inspect and identify its problem 1.2 Check and inspect the defects of the breech mechanism. 1.3 File worn out striker assembly. 1.4 Remove allowable dents, nicks, scratches by filing. 1.5 Replace damaged striker 1.6 Replace damaged firing pin and firing spring. 1.7 Remove dents, nicks scratches from firing pin housing 1.8.Remove dents, nicks, and scratches from extractor and the housing. 1.9. Replace damaged extractor and spring. 2.0. Replace worn out pins, cocking cam, breech block, shaped recess and lugs. |
| 2. Repair Stock and hand guard assembly | 2.1. Disassemble, Check and inspect the problem of the stock and hand guard assembly. 2.2. Cracked and damaged stock and hand guard assembly is maintained using the repair manual specifications. 2.3. Badly damaged stock and hand guard assembly is repaired |
| 3. Repair Operating rod with piston | 3.1. Disassemble, Check and inspect the problem that occurred on the rod and piston. 3.2. Dents, scratches, on the piston head are removed slightly using an appropriate material. 3.3. Smoke, deposition from the piston rod and cylinder are removed. 3.4. Badly bend and damaged rod are rejected for farther maintenance. 3.5. Assemble and check its malfunction. |
| 4.Maintain Bolt assembly | 4.1. Disassemble, Check and inspect the problem that occurred on the bolt assembly 4.2.Disassemble and Check ,Inspect firing pin extrusion as per specifications 4.3 Remove dents, nicks scratches from firing pin housing and replace damaged firing pin and firing spring 4.4 Check and inspect extractor proper functioning 4.5 Remove dents, nicks, and scratches from extractor and the housing. 4.6. Damaged pins springs are replaced. 4.7. Assembled and check malfunction of the bolt assembly. |
| 5. Repair Barrel and Receiver group | 5.1 Disassemble barrel and identify its problems 5.2 Check and inspect the bore visually 5.3. Measure width and depth using inappropriate tool. 5.4. Damaged lands/rifles beyond the limit are rejected 5.5 Inspect the degree of nicks. 5.6. Slight nicks on barrel are removed by removing minimum layer of material 5.7. Rusts and powder gases disposition is removed from the bore using an appropriate material. 5.8 Use chemicals for farther cleaning. 5.9. Cracked barrel is rejected 5.10 Bend sears are adjusted, damaged sears are replaced. 5.11. Dents, scratches, bulges on slide stops are removed using an appropriate. 5.12.Assemble the barrel and receiver and check its functionality |
| 6.Maintain Rifle bipod | 6.1.Dis assemble the bipod and check and inspect. 6.2. Slight bends are adjusted, using an appropriate tool 6.3.Broken bipod is welded. 6.4.Damage springs are replaced. 6.5. After maintenance assemble the bipod and check its functionality |
| 7.Maintain Rifle tripod | 7.1. Disassemble the tripod and check and inspect. 7.2. Slight bends are adjusted, using an appropriate tool 7.3. Broken tripod is welded. 7.4.Damage springs are replaced. 7.5. After maintenance assemble the tripod and check its functionality |

| Variables | # Range |
| --- | --- |
| Firing Mechanism | May include but not limited to: - PKM Firing mechanism - PKT Firing mechanism |
| Breech Mechanism | May include but not limited to: - PKM Breeching mechanism - PKT Breeching mechanism |
| Barrel | May include but not limited to: - PKM Barrel - PKT Barrel |
| Occupational Health & Safety (OHS) | May include but not limited to: - Using of relevant Uniforms and protective when necessary - Using Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones , gloves etc, - Maintaining fire extinguishers and protection tools at appropriate place - Following Occupational health and safety procedures designated for the task - Checking and fulfilling required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: - SPTA - Welding machines - Belching chemicals(anti rust) - Manufacturing machines - Hammer, mallets, wire brush etc - Measuring tools - Wire brush, sand paper - Wool and cloth |
| Types and Sources of Information | May include but not limited to: - Information source may include variable written and graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate: - Described Maintain procedures and precautions of barrel with receiver assembly - Followed work instructions and procedures to determine job requirements - Identify the problems on the firing mechanism - Describe the maintenance procedure of bolt assembly, stock and hand guard assembly. - Maintained , piston rod and (cylinder) - Identified and taken measure for gas leakages - demonstrated safe work procedures - Identified proper working tools - Cleaned up work area and maintained equipment |
| Underpinning knowledge and attitude | Demonstrates Knowledge of: - Basic manufacturing - Basic designing and specification - General knowledge of maintenance - Work ethics - Basic Metallurgy - Technical drawing - Fundamentals small arms - Fundamentals of optics - Basic mathematics knowledge - Basic writing and speaking knowledge - General knowledge of material management and shop - Basic sheet metal work - Tactical employment of PKM and PKT - Ballistics data of PKM and PKT - Working principle of the automatic mechanism |
| Underpinning skill | Demonstrates Knowledge of: - Stripping the guns in detail - Check the status of every part - Repair Firing Mechanism - Repair Stock and hand guard assembly - Repair Barrel and Receiver group - Repair Operating rod - Assemble and - check every mechanism |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Perform Painting Operation |
| Unit Code | IND SHA2 12 0616 |
| Unit Descriptor | This unit covers the skills and knowledge required in understanding the body painting operations. |

| Elements | Performance Criteria |
| --- | --- |
| - Perform Preparation spray painting materials and equipment | - Types of painting materials are identified - Preparing paint (paint mixing) - Types ,parts and perform Troubleshooting spray gun are identified - Usage of sanding materials and equipment is identified - Work is compressed with air supply - Perform Paint mixing techniques following Occupational Health & Safety (OHS) - Paint drying methods |
| - Prepare spray booths and paint drying equipment | - Prepare Spray booths - Different Types of spray booths and drying equipment are identified - Air filtration system is repaired - Spray booth maintenance is performed - Clean up work area and maintain equipment |

| Variable | Range |
| --- | --- |
| Materials and equipment | May include but not limited to: - SPTA - Sand paper - Scraper - Spray gun ( machine ) - Air compressor - Files - Belching chemicals(anti rust) - Manufacturing machines - Hammer, wire brush etc - Measuring tools |
| OHS may include: | - Use of relevant Uniforms and protective when necessary - Use of Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones etc, - Maintain fire extinguishers and protection tools at appropriate place - Follow Occupational health and safety procedures designated for the task - Check and fulfill required safety devices before starting operation |
| Types and Sources of Information | May include but not limited to: - Information source may include variable written and graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Painting diagrams, charts - Painting catalogue/specification manual. - Painting technique manuals - Design specification manual - Repair request documents - Records and reports, Internet and Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires knowledge and skills in: - procedures and precautions during paint preparation - identifying and inspecting material for repairing and painting - demonstrating safe working procedures - describing procedures during preparation of parts before painting |
| Underpinning Knowledge and Attitudes | Look for evidence that confirms knowledge of: - Basic painting - Chemicals - Designing and specification - Maintenance - Material management and shop organization - Input/output devices |
| Underpinning Skills | Demonstrates skills to: - Prepare the part to be paint by sanding - Apply every procedure before painting - Mix the dye to get the right colour - Apply the paint using the right method |
| Resource Implications | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Participate in Workplace Communication |
| Unit Code | IND SHA2 13 0616 |
| Unit Descriptor | This unit covers the knowledge, skills and attitudes required to gather, interpret and convey information in response to workplace requirements. |

| Elements | Performance Criteria |
| --- | --- |
| - Obtain and convey workplace information | - Specific and relevant information is accessed from appropriate sources. - Effective questioning, active listening and speaking skills are used to gather and convey information. - Appropriate medium is used to transfer information and ideas. - Appropriate non- verbal communication is used. - Appropriate lines of communication with supervisors and colleagues are identified and followed. - Defined workplace procedures for the location and storage of information are used. - Personal interaction is carried out clearly and concisely. |
| - Participate in workplace meetings and discussions | - Team meetings are attended on time. - Own opinions are clearly expressed and those of others are listened to without interruption. - Meeting inputs are made consistent with the meeting purpose and protocols established. - Workplace interactions are conducted in a courteous manner. - Questions about simple routine workplace procedures and matters concerning working conditions of employment are asked and responded. - Meetings outcomes are interpreted and implemented. |
| - Complete relevant work related documents | - Range of forms relating to conditions of employment is completed accurately and legibly. - Workplace data is recorded on standard workplace forms and documents. - Basic mathematical processes are used for routine calculations. - Errors in recording information on forms/ documents are identified and properly acted upon. - Reporting requirements to supervisor are completed according to organizational guidelines. |

| Variable | Range |
| --- | --- |
| Appropriate sources | May include but not limited to: - Team members - Suppliers - Trade personnel - Local government and Industry bodies |
| Medium | May include but not limited to: - Memorandum - Circular - Notice - Information discussion - Follow-up or verbal instructions & Face to face communication |
| Storage | May include but not limited to: - Manual filing and computer-based filing systems |
| Protocols | May include but not limited to: - Observing meeting - Compliance with meeting decisions - Obeying meeting instructions |
| Workplace interactions | May include but not limited to: - Face to face - Telephone - Electronic and two way radio - Written including electronic, memos, instruction and forms, non-verbal including gestures, signals, signs and diagrams |
| Forms | May include but not limited to: - Personnel forms, telephone message forms, safety reports |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competency | Demonstrates skills and knowledge to: - Prepare written communication following standard format of the organization - Access information using communication equipment - Make use of relevant terms as an aid to transfer information effectively - Convey information effectively adopting the formal or informal communication |
| Underpinning Knowledge and Attitudes | Demonstrate knowledge of: - Effective communication - Different modes of communication - Written communication - Organizational policies - Communication procedures and systems - Technology relevant to the enterprise and the individual’s work responsibilities |
| Underpinning Skills | Demonstrate skills to: - Follow simple spoken language - Perform routine workplace duties following simple written notices - Participate in workplace meetings and discussions - Complete work related documents - Estimate, calculate and record routine workplace measures - Do basic mathematical processes of addition, subtraction, division and multiplication - relate to people of social range in the workplace - Gather and provide information in response to workplace Requirements |
| Resource Implications | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Work in Team Environment |
| Unit Code | IND SHA2 14 0616 |
| Unit Descriptor | This unit covers the skills, knowledge and attitudes to identify role and responsibility as a member of a team. |

| Elements | Performance Criteria |
| --- | --- |
| - Describe team role and scope | - The role and objective of the team are identified from available sources of information. - Team parameters, reporting relationships and responsibilities are identified from team discussions and appropriate external sources. |
| - Identify own role and responsibility within team | - Individual role and responsibilities within the team environment are identified. - Roles and responsibility of other team members are identified and recognized. - Reporting relationships within team and external to team are identified. |
| - Work as a team member | - Effective and appropriate forms of communications are used and interactions undertaken with team members who contribute to known team activities and objectives. - Effective and appropriate contributions are made to complement team activities and objectives, based on individual skills and competencies and workplace context. - Protocols are observed in reporting using standard operating procedures. - Contribution is made to the development of team work plans based on an understanding of team’s role and objectives and individual competencies of the members. |

| Variable | Range |
| --- | --- |
| Role and objective of team | May include but not limited to: - Work activities in a team environment with enterprise or specific sector - Limited discretion, initiative and judgment maybe demonstrated on the job, either individually or in a team environment |
| Sources of information | May include but not limited to: - Standard operating and/or other workplace procedures - Job procedures - Machine/equipment manufacturer’s specifications and instructions - Organizational or external personnel - Client/supplier instructions - Quality standards - OHS and environmental standards |
| Workplace context | May include but not limited to: - Work procedures and practices - Conditions of work environments - Legislation and industrial agreements - Standard work practice including the storage, safe handling and disposal of chemicals - Safety, environmental, housekeeping and quality guidelines |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Demonstrates skills and knowledge to: - Operate in a team to complete workplace activity - Work effectively with others - Convey information in written or oral form - Select and use appropriate workplace language - Follow designated work plan for the job - Report outcomes |
| Underpinning Knowledge and Attitude | Demonstrate knowledge of: - Communication process - Team structure - Team roles - Group planning and decision making |
| Underpinning Skills | Demonstrate skills to: - Communicate appropriately, consistent with the culture of the workplace |
| Resource Implications | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Develop Business Practice |
| Unit Code | IND SHA2 15 0616 |
| Unit Descriptor | This unit covers knowledge, skills and attitude required to establish a business operation from a planned concept. It includes researching the feasibility of establishing a business operation, planning the setting up of the business, implementing the plan and reviewing operations once commenced, customer handling, developing and maintaining business relationships. |

| Elements | Performance Criteria |
| --- | --- |
| - Identify business opportunities and business skills | - The concept of paradigm shift and means of divergent thinking are elaborated and strategies to look beyond the boundaries are discussed. - Unusual business opportunities are identified. - Feasibility on business skills and personal attributes is assessed and matched against those perceived as necessary for a particular business opportunity. - New behavior on how problems can be the pivotal source of business opportunity is elaborated and experience taken. - Assistance sought with feasibility study of specialist and relevant parties is discussed, as required. - Impact of emerging or changing technology, including e-commerce, on business operations is evaluated. - Practicability of business opportunity is assessed in line with perceived business risks, returns sought, personal preferences and resources available. - Business plan is revised in accordance with the identified opportunities. |
| - Plan for the establishment of business operation | - Organizational structure and operations are determined and documented. - Procedures are developed and documented to guide operations. - Financial backing is secured for business operation. - Business legal and regulatory requirements are identified and compiled. - Human and physical resources required to commence business operation are determined. - Recruitment and procurement strategies are developed. |
| - Implement Business Development Plan | - Physical and human resources are obtained to implement business operation. - Operational unit is established to support and coordinate business operation. - Simulations on the development plan are well discussed and understood. - Implementation manual is discussed and understood. - Marketing the business operation is undertaken. - Monitoring process is developed and implemented for managing operation. - Legal documents are carefully maintained and relevant records kept and updated to ensure validity and accessibility. - Contractual procurement rights for goods and services including contracts with relevant people are negotiated and secured as required in accordance with the business plan. - Options for leasing/ownership of business premises are identified and contractual arrangements completed in accordance with the business plan. |
| - Review implementation process and take corrective measures | - Review process is developed and implemented for implementation of business operation. - Improvements in business operation and associated management process are identified. - Identified improvements are implemented and monitored for effectiveness. |
| - Establish contact with customers and clarify needs of customer | - Persuasion strategies are developed and discussed. - Welcoming customer environment is maintained and Customer is greeted warmly according to enterprise policies and procedures. - Information is provided to satisfy customer needs. - Information on customers and service history is gathered for analysis. - Customer data is maintained to ensure database relevance and currency. - Customer needs are accurately assessed against the products/services of the enterprise. - Customer details are documented clearly and accurately in required format. - Negotiations are conducted in a business-like and professional manner. - Benefits for all parties are maximized in the negotiation through use of established techniques and in the context of establishing long term relationships. - The results of negotiations are communicated to appropriate colleagues and stakeholders within appropriate timeframes. - Opportunities to maintain regular contact with customers are identified and taken-up. |
| - Develop and Maintain Business Relationship | - Features and benefits of products/services provided by the enterprise are described/ recommended to meet customer needs. - Alternative sources of information/advice are discussed with the customer. - Information needed is pro-actively sought, reviewed and acted upon to maintain sound business relationships. - Agreements are honored within the scope of individual responsibility. - Adjustments to agreements are made in consultation with the customer and information shared with appropriate colleagues. - Relationships are nurtured through regular contact and use of effective interpersonal and communication styles. |

| Variable | Range |
| --- | --- |
| Unusual Business opportunities | May include but not limited to: - Public holidays - Ceremonies - Natural disaster - Campaigns |
| Business opportunities | May include but not limited to: - Expected financial viability - Skills of operator - Amount and types of finance available - Returns expected or required by owners - Likely return on investment - finance required - Lifestyle issues |
| Business skills and personal attributes | May include but not limited to: - Technical and/ or specialist skills - Managerial skills - Entrepreneurial skills - Taking calculated risk skills - Willingness to take calculated risks - Willingness to work under pressure |
| Specialist and relevant parties | May include but not limited to: - Chamber of commerce - Financial planners and financial institution representatives, business planning specialists and marketing specialists - Accountants - Lawyers and providers of legal advice - Government agencies - Industry/trade associations - Online gateways - Business brokers/business consultants |
| Business risks | May include but not limited to: - Occupational health and safety - Environmental risks - Relevant legislative requirements - Security of investment - Market competition - Security of premises/location - Supply and demand - Resources available |
| Human and physical resources | May include but not limited to: - Software and hardware - Office premises and equipment - Communications equipment - Specialist services through outsourcing, contracting and consultancy - Staff - Vehicles |
| Operational unit | May include but not limited to: - different departments, sections, teams, divisions, etc. staffed with required personnel and equipped to service and support business |
| Legal documents | May include but not limited to: - Partnership agreements, constitution documents, statutory books for companies (register of members, register of directors and minute books), certificate of Incorporation, franchise agreements and financial documentation, appropriate software for financial records - Occupational Health Safety (OHS) - Recordkeeping including personnel, financial, taxation, and environmental |
| Contracts with relevant people | May include but not limited to: - business owners, suppliers, employees, agents, land owners, distributors, customers or any person with whom the business has, or seeks to have, a performance-based relationship |
| Negotiation techniques | May include but not limited to: - Identification of goals, limits - Clarification of needs of all parties - Listening and questioning - Non-verbal communication techniques - Appropriate language and situation - Bargaining - Developing options - Appropriate cultural behavior - Confirming agreements |
| Opportunities to maintain regular contact | to maintain regular contact with customers may include: - Informal social occasions - Ceremonies - Exhibitions - Industry functions - Association membership - Co-operative promotions - Program of regular telephone contact |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Demonstrates knowledge and skills in: - that a business operation has been planned and implemented from initial research of feasibility of the business and completion of the plan, through implementing the plan and commencing operations - the ability to evaluate the results of research and assess the likely viability and practicability of a business opportunity, taking into account the current business/market climate and resources available - treating customers in a courteous and professional manner - building and maintaining relationships to achieve successful business outcomes |
| Underpinning Knowledge and Attitudes | Demonstrate knowledge of: - Paradigm shift - Unusual business opportunities - Feasibility study - Business structure - Federal and regional government legislative requirements affecting business operations, especially in regard to Occupational Health and Safety (OHS), Equal Employment Opportunity (EEO), industrial relations and anti-discrimination - Procurement and recruitment strategy - Operational unit - Monitoring process - Business systems and operations - Relevant marketing, management, sales and financial concepts - Options for financing - Business premises and ownership - Lease - Methods for researching business opportunities - Methods of identifying relevant specialist services to complement the business - Advertising and promotion - Distribution and logistics - Terms and conditions in contractual agreement - Record keeping duties - Operational factors relating to the business (provision of professional services, products) - Customer need assessment - Source of information - Operational knowledge of enterprise policies and procedures in regard to: - customer service - dealing with difficult customers - maintenance of customer databases - allocated duties/responsibilities - General knowledge of the range of enterprise merchandise and services, location of telephone extensions and departments/sections - Basic operational knowledge of industry/workplace codes of practice in relation to customer service - negotiation and communication techniques appropriate to negotiations that may be of significant commercial value |
| Underpinning Skills | Demonstrate skills of: - Hunting and exploiting unusual business opportunities - Interpreting legal requirements, company policies and procedures and immediate, day-to-day demands - Conducting feasibility study - Developing new behavior - Using technology - Marketing skills - Business planning skills - Entrepreneurial skills - Time management skills - Customer handling skills - Communication skills including questioning, clarifying, reporting, and giving and receiving constructive feedback - Technical and analytical skills to interpret business documents, reports and financial statements and projections - Ability to relate to people from a range of social, cultural and ethnic backgrounds and physical and mental abilities - Problem solving skills to develop contingency plans - Using computers and software packages to record and manage data and to produce reports - Interpreting business information, numeracy skills for data analysis to aid research - Negotiation to conduct business activities - Research to identify a business opportunity and to conduct a feasibility study - Analytical skills to assess personal attributes and to identify business risks - Observation skills for identifying appropriate people, resources and to monitor work - Persuasion and networking skills - Welcoming customers - Information seeking skills to collect, organize and understand information related to collating and analyzing customer information to identify needs - Establish diagnostic processes which identify and recommend improvements to customer service |
| Resource Implications | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level II |  |
| --- | --- |
| Unit Title | Standardize and Sustain 3S |
| Unit Code | IND SHA2 16 0616 |
| Unit Descriptor | This unit of competence covers the knowledge, skills and attitudes required by worker to standardize and sustain 3S to his/her workplace. It covers responsibility for the day- to-day operations of the workplace and ensuring that continuous improvements of Kaizen elements are initiated and institutionalized. |

| Elements | Performance Criteria |
| --- | --- |
| 1. Prepare for work. | - Work instructions are used to determine job requirements, including method, material and equipment. - Job specifications are read and interpreted following working manual. - OHS requirements, including dust and fume collection, breathing apparatus and eye and ear personal protection needs are observed throughout the work. - Safety equipment and tools are identified and checked for safe and effective operation. - Tools and equipment are prepared and used to implement 3S. |
| - Standardize 3S. | - Plan is prepared and used to standardize 3S activities. - Tools and techniques to standardize 3S are prepared and implemented based on relevant procedures. - Checklists are followed for standardize activities and reported to relevant personnel. - The workplace is kept to the specified standard. - Problems are avoided by standardizing activities. |
| - Sustain 3S. | - Plan is prepared and followed to standardize 3S activities. - Tools and techniques to sustain 3S are discussed, prepared and implemented based on relevant procedures. - Workplace is inspected regularly for compliance to specified standard and sustainability of 3S techniques. - Workplace is cleaned up after completion of job and before commencing next job or end of shift. - Situations are identified where compliance to standards is unlikely and actions specified in procedures are taken. - Improvements are recommended to lift the level of compliance in the workplace. - Checklists are followed to sustain activities and report to relevant personnel. - Problems are avoided by sustaining activities. |

| Variable | Range |
| --- | --- |
| OHS requirements | May include but not limited to: - Are to be in accordance with legislation/ regulations/codes of practice and enterprise safety policies and procedures. This may include protective clothing and equipment, use of tooling and equipment, workplace environment and safety, handling of material, use of fire fighting equipment, enterprise first aid, hazard control and hazardous materials and substances. - Personal protective equipment is to include that prescribed under legislation/regulations/codes of practice and workplace policies and practices. - Safe operating procedures are to include, but are not limited to the conduct of operational risk assessment and treatments associated with workplace organization. - Emergency procedures related to this unit are to include but may not be limited to emergency shutdown and stopping of equipment, extinguishing fires, enterprise first aid requirements and site evacuation. |
| Safety equipment and tools | May include but not limited to: - dust masks / goggles - glove - working cloth - first aid and safety shoes |
| Tools and equipment | May include but not limited to: - paint - hook - sticker - signboard - nails - shelves - chip wood - sponge - broom - pencil - shadow board/ tools board |
| Tools and techniques | May include but not limited to: - 5S Job Cycle Charts - Visual 5S - The Five Minute 5S - Standardization level checklist - 5S checklist - The five Whys and one How approach(5W1H) - Suspension - Incorporation and Use Elimination |
| Relevant procedures | May include but not limited to: - Assign 3S responsibilities - Integrate 3S duties into regular work duties - Check on 3S maintenance level - OHS measures such as signage, symbols / coding and labeling of workplace and equipment - Creating conditions to sustain your plans - Roles in implementation |
| Reporting | May include but not limited to: - verbal responses - data entry into enterprise database - brief written reports using enterprise report formats |
| Relevant personnel | May include but not limited to: - supervisors, managers and quality managers - administrative, laboratory and production personnel - internal/external contractors, customers and suppliers |
| Tools and techniques | May include but not limited to: - 5S slogans - 5S posters - 5S photo exhibits and storyboards - 5S newsletter - 5S maps - 5S pocket manuals - 5S department/benchmarking tours - 5S months - 5S audit - Awarding system - Big cleaning day - Patrolling system may include: - Top management Patrol - 5S Committee members and Promotion office Patrol - Mutual patrol - Self-patrol - Checklist patrol - Camera patrol |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Demonstrates skills and knowledge to: - Discuss the relationship between Kaizen elements. - Standardize and sustain 3S activities by applying appropriate tools and techniques. |
| Underpinning Knowledge and Attitudes | Demonstrates knowledge of: - Elements of Kaizen - Ways to improve Kaizen elements - Benefits of improving kaizen elements - Relationship between Kaizen elements - The fourth pillar of 5S - Benefits of standardizing and sustaining 3S - Procedures for standardizing and sustaining 3S activities - Tools and techniques to sustain 3S - Relevant Occupational Health and Safety (OHS) and environment requirements - Plan and report - Method of communication |
| Underpinning Skills | Demonstrates skills of: - improving Kaizen elements by applying 5S - standardizing and sustaining procedures and techniques to avoid problems - technical drawing - procedures to standardizing 3S activities - analyzing and preparing shop layout of the workplace - standardizing and sustaining checklists - preparing and implementing tools and techniques to sustain 3S - working with others - reading and interpreting documents - observing situations - solving problems by applying 5S - communication skills - preparing labels, slogans, etc. - gathering evidence by using different means - using Kaizen board properly in accordance the procedure - reporting activities and results using report formats |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

NTQF Level III

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Read and Interpret Machine Drawing |
| Unit Code | IND SHA3 01 0616 |
| Unit Descriptor | This unit is designed to equip trainees with machine drawing. It contains both the theoretical part and practical activities the trainee should pass through to acquire the basic competence (skill, knowledge and attitude) in accomplishing the needs. |

| Elements | Performance Criteria |
| --- | --- |
| - Perform different dimensioning techniques | - Methods of dimensioning a drawing are performed with sources of information. - Drawing to scale is performed. |
| - Perform drawing metric screw threads | - Screw threads representation is performed on a drawing. - Assembly drawing is identified with threaded and non-threaded elements. |
| - Perform drawing of pipes | - Representing pipe assembly is performed on drawing. - Repressing square pipe assemble is performed on drawing. - Drawing tools and equipment are performed. |
| - Read ,interpret and draw machine elements | - Drawing of keys, machine pins, rivets, pulleys are performed. - Gear representing techniques are performed on a drawing. - Bearings and springs representation techniques are performed on a drawing. - Lubricating organs representing techniques are performed on a drawing. - Hydro-mechanical devices representing techniques on a drawing is performed. - Pneumatic devices representing techniques on a drawing is performed. - Electrical drawing representing techniques are performed on a drawing. |
| - Prepare assembly drawings | - Preparation assembly drawings of different parts are performed. - Preparation assemble drawing of machine parts are performed. |
| - Read, interpret and draw geometric tolerance | - Represents tolerances of form, profile, run out are read, interpreted and represented. - Limits and fits on drawing are represented. |

| Variable | Range |
| --- | --- |
| Sources of information | - May include but not limited to: - Information source written and graphical instructions, work bulletins, data sheet, diagrams or sketches include: - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |
| Tools and equipment | - May include but not limited to: - Set square. - Rubber(eraser) - Drawing pen - Different drawing papers - Ruler - Drawing table - brush - different Measuring tools - different machines |
| Occupational Health & Safety (OHS) | - May include but not limited to: - Use of relevant Uniforms and protective when necessary - Follow Occupational health and safety procedures designated for the task - Check and fulfill required safety devices before starting operation |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | - Demonstrate knowledge and skills to: - Safety procedures during machine drawing operation. - Perform drawings of different machine elements, like gears, pulleys, keys, pins, threads, etc |
| Underpinning Knowledge and Attitudes | - Demonstrate knowledge of: - Material management - Work organization and planning processes - Basic Metallurgy - Technical drawing |
| Underpinning Skills | - Demonstrate skills of: - checking the drawing against job requirements/related equipment in accordance with standard operating procedures - confirming the drawing version as being current in accordance with standard operating procedures - where appropriate, obtaining the current version of the drawing in accordance with standard operating procedures - reading, interpreting information on the drawing, written job instructions, specifications, standard operating procedures, charts, lists and other applicable reference documents - checking and clarifying task related information undertaking numerical operations, geometry and calculations/formulae within the scope of this unit |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Set Assembly Stations |
| Unit Code | IND SHA3 02 0616 |
| Unit Descriptor | This unit covers setting up, adjusting and testing assembly stations according to defined procedures. |

| Elements | Performance Criteria |
| --- | --- |
| 1. Identify job requirements | 1.1.Prepare the assembly stations 1.2. Setting requirements are correctly identified from job sheets/instructions. |
| 2. Select and use a range of hand tools and equipment | 2.1.Prepair the hand tools and equipments 2.2. Hand tools and equipment for setting assembly stations are used in a safe manner, according to instructions, standard operating procedures and any legislative requirements. |
| 3. Set assembly stations | 3.1. Assembly stations are set up for a range of processes and operations according to defined procedures. 3.2. Safe work practices are observed and implemented. 3.3. Assembly stations are adjusted to specifications and operational requirements. 3.4. Assembly stations are tested for correct operation. |
| 4. Maintain equipment | 4.1. Routine maintenance is carried out to standard operating procedures. 4.2. Worn or damaged components are identified and changed. |

| Variable | Range |
| --- | --- |
| Job sheets/instructions | May include but not limited to: - Verbal and written job instructions, specifications, standard operating procedures and assembly drawings within the scope of this unit |
| Assembly station | May be used for operations such as riveting, pressing, screwing, tensioning etc. and processes such as testing, gluing, identification, numbering or simple hot stamping etc. |
| Set up | May include to ensure that appropriate jigs, fixtures, die sets, stores and tooling etc. are in place as required to meet the production order or schedules |
| Processes and operations | May include to rivet, press, screw, tension, test, glue, identification, numbering or simple hot stamping of components and assemblies |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessors must be satisfied that the candidate can competently and consistently perform all elements of the unit as specified by the criteria, including required knowledge, and be capable of applying the competency in new and different situations and contexts. |
| Underpinning Knowledge | Demonstrates knowledge of: - procedures to be followed in setting up assembly stations - safety hazards associated with the assembly station and/or its setting up - specifications applicable to the assembly station - effect of various adjustments that can be made to the assembly station - routine maintenance requirements - effect of worn or damaged components on the operational requirements and specifications of the assembly station - use and application of personal protective equipment - safe work practices and procedures |
| Underpinning Skills | Demonstrates skills of: - reading and interpreting routine information on written job instructions, specifications and standard operating procedures. May include drawings for setting assembly stations - following oral instructions - testing and checking assembly station and outputs - identifying worn and/or damaged components |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Perform Precision Assembly |
| Unit code | IND SHA3 03 0616 |
| Unit Descriptor | This unit covers assembling and testing complex engineering components and mechanical assemblies in a production, manufacturing environment. It focuses on perform precision assembling in fabrication and assembly of metals. |

| Elements | Performance Criteria |
| --- | --- |
| - Prepare for work | - Job sheet or equivalent instructions are interpreted correctly. - All components/sub-assemblies are checked against job sheet, assembly list or equivalent instructions. - Fitting requirements and sequential assembly planning are carried out where applicable. - Tools, equipment and components/sub-assemblies are selected to meet job requirements. - OHS procedures are observed through the process in this unit. |
| - Assemble engineering components | - Defective or faulty components/sub-assemblies are identified and processed according to standard operating procedure. - Components/sub components are correctly prepared for assembly. - Techniques and principles appropriate to the job requirements are applied in assembly activity. - Records/inputs are accurately maintained or processed. - Components of assembly are fitted to ensure correct positioning and conformance with specifications. - Assembly is tested to ensure that components interface/interact according to operational specifications. |
| - Adjust mechanical assemblies | 3.1 Final adjustments are performed on assembly to ensure alignment with operational specifications. 3.2 Faulty assemblies are identified for rework or when the fault is outside the scope of the workstation, processed according to standard operating procedure. 3.3 The assembly is correctly marked / tagged / identified. |
| - Protect assembly from damage | 4.1 Components and/or assembly are handled according to standard operating procedures and in a manner least likely to cause damage. 4.2 Components and/or assembly are stored according to standard operating procedures and in a manner least likely to cause damage |

| Variable | Range |
| --- | --- |
| Components/sub-assemblies | May include but not limited to: - parts that make up the sub-assembly and components |
| Tools and equipment | May include but not limited to: - hand, power and portable tools, and equipment - measuring equipment |
| Occupational Health and Safety (OHS) | OHS requirements are to be in accordance with Federal legislation and regulations and may include but not limited to: - protective clothing and equipment (include that prescribed under legislation, regulation and workplace policies) - safe use of tools and equipment and materials - workplace environment and safety and hazard control - use of firefighting and first aid equipment |
| Technique | May include but not limited to: Assembly methods and correct use of hand tools |
| Assembly testing | May include but not limited to: Testing and checking to specification of assembled product |
| Adjustment | May include but not limited to: Clearances, mesh, tension, level, alignment etc. using predetermined standards of quality and safety |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate: - correctly interpreted instructions in the job sheet - carried-out sequential assembly planning and fitting requirements - selected tools, equipment and components/sub-assemblies based on job requirements - prepared and assembled components/sub-components - applied correct techniques and principles during assembly activity - fitted components of assembly to ensure correct positioning - tested assembly to ensure that components are interface/interact according to operational specifications - adjusted mechanical assemblies if required - marked or tagged assembly without faults and stored in designated place |
| Underpinning knowledge and Attitudes | Demonstrates knowledge of: - job sheet /instruction interpretation - specifications and types of tools, equipment and materials - criteria in selecting /determining components/sub-components - principles and techniques in assembling components/sub-components - fitting requirements - sequence in which the tasks are to be performed - required action for test and checks, and non-conformance - sources of the component/sub-assemblies - criteria in determining good component assemblies from defective component assemblies - damage to components and/or assemblies through the use of inappropriate handling and/or unsafe storage procedures - relevant record keeping requirements - safe work practices and procedures - hazards and control measures associated with precision assembly |
| Underpinning Skills | Demonstrates skills of: - operating relevant tools and equipment - testing and checking assemblies - preparing a sequential assembly plan - selecting and sourcing appropriate tools, components and sub-assemblies - using tools appropriately - identifying faulty components - following job and oral instructions - performing relevant record keeping requirements - storing assemblies without damage |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Repair of Medium Machine Guns |
| Unit Code | IND SHA3 04 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding servicing, maintenance as well repairing different parts and mechanisms of medium machine guns. |

| Elements | Performance Criteria |
| --- | --- |
| 1.Repair Barrel | 1.1Disassemble barrel and identify its problems 1.2 Check and inspect the bore visually 1.3. Measure width and depth using appropriate tool. 1.4. Damaged lands/rifles beyond the limit are rejected 1.5 Inspect the degree of nicks. 1.6. Slight nicks on barrel are removed by removing minimum layer of material 1.7. Rusts and powder gases disposition is removed from the bore using an appropriate material. 1.8 Cracked barrel is rejected |
| 2. Repair Receiver | 2.1. Inspect receiver and identify its problems 2.2. Bend sears are adjusted, damaged sears are replaced. 2.3. Dents, scratches, bulges on the guide ways are removed using sand paper and file. 2.4. Remove old lubricant and replace with the new and appropriate lubricant 2.5. Finally check its functionality. |
| 3. Repair Recoiling Mechanism | 3.1. Disassemble recoil mechanism and Correct information is accessed and interpreted from Manufacturer’s and designer’s specifications. 3.2 Springs are manufactured using approved method based on workplace guidelines. 3.3 Quality of springs is checked and tested according to the standard working procedures. 3.4 Broken or weak springs are replaced correctly on its right position according to manufacturer’s specifications. 3.5. Dents, scratches, bulges on spring guide rod are removed. 3.6. Assembled and check malfunction of the recoil mechanism |
| 4.Repair Feed Mechanism | 4.1. Disassemble feed mechanism and Correct information is accessed and interpreted from Manufacturer’s and designer’s specifications. 4.2. Damaged link ejectors are hammered. 4.3. Broken or torn ejectors are replaced. 4..4. Damaged pins and springs are replaced |
| 5. Repair Trigger Mechanism | 5.1. Disassemble trigger mechanism and Correct information is accessed and interpreted from Manufacturer’s and designer’s specifications. 5.2. Cracked and broken trigger guard is welded. 5.3. Brocken trigger lugs are welded and re-machined slightly to achieve its right size and dimension. 5.4. damaged pins and springs are replaced 5.4. Badly damaged trigger guard, trigger lugs, springs and pins are rejected. 5.5.Assembled and check malfunction of the trigger mechanism |
| 6.Repair Back Plate Mechanism | 6.1 Disassemble back plate mechanism and Correct information is accessed and interpreted from Manufacturer’s and designer’s specifications. 6.2. Damaged and broken spring is replaced 6.3. Slight damaged teeth and key of the plate is adjusted using an appropriate tool 6.4. Badly damaged back plat is replaced. 6.5. Assembled and check malfunction of the back plate mechanism |
| 7 Repair Cartridge box with belt | 7.1. Cracked and broken Cartridge box is welded 7.2.Bend lugs are adjusted by filing and sand paper 7.3. Damaged springs are replaced 7.4.Damaged pins are replaced 7.5. Cracked cartridge box are welded 7.6. Rust and corroded boxes are cleaned and painted |
| 8. Repair Medium Machine Guns Mount | 8.1. Damaged and broken Mount is replaced 8.2. Damaged tip of the leg are maintained by welding 8.3.Scraches and dents are filed on the trunnion and damaged trunnion are replaced 8.4. Damaged bolts are replaced 8.5. Brocken lock springs are replaced 8.6. Corrosive and rust mount are cleaned and painted. |
| 9. perform Zeroing | 9.1. Aligning optical sight 9.2. Adjust line of bore and line of sight 9.3.Adjust line of bore and line of sight 9.4.Adjust line of open sight and line of bore 9.5. Adjust line of anti air craft sight and line of bore 9.6. Lost nuts on the wheel are replaced |

| Variables | # Range |
| --- | --- |
| Barrel | May include but not limited to: - Barrel of 12.7 Medium Machine Gun |
| Receiver | May include but not limited to: - receiver of12.7 Medium Machine Gun |
| Recoil mechanism | May include but not limited to: - recoil mechanism of12.7 Medium Machine Gun |
| Trigger mechanism | May include but not limited to: - trigger mechanism of12.7 Medium Machine Gun |
| Feed mechanism | May include but not limited to: - feed mechanism of12.7 Medium Machine Gun |
| Occupational Health & Safety (OHS) | May include but not limited to: - Using relevant Uniforms and protective when necessary - Using Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones etc, - Maintaining fire extinguishers and protection tools at appropriate place - Following Occupational health and safety procedures designated for the task - Checking and fulfilling required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: - SPTA - Lifting machines - Scraper - Welding machine - Files - Bleach chemicals(anti rust) - Manufacturing machines - Hammer, wire brush etc - Measuring tools |
| Types and Sources of Information | May include but not limited to: - graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that: - Identified and inspected material for repair - Described principles and procedures for Identify defects of the Barrel, Receiver, the Recoiling Mechanism, Feeding Mechanism, Trigger Mechanism, Back Plate Mechanism, Cartridge box with belt, assemble every part of the gun after, maintenance - Demonstrated safe working procedures - Replaced damaged valves and collars, broken and weak springs. - Cleaned up work area and maintained gun part mechanism. |
| Underpinning Knowledge and Attitude | Demonstrates Knowledge of: - Basic manufacturing - Basic designing and specification - General knowledge of maintenance - Work ethics - Basic Metallurgy - Technical drawing - Fundamentals small arms - Fundamentals of optics - Basic mathematics knowledge - Basic writing and speaking knowledge - General knowledge of material management and shop - Tactical behavior of medium machine guns. - Ballistics characteristics of 12.7mm medium machine guns - Theory of the Fire control system - The functioning of the different mechanisms |
| Underpinning skill | Demonstrates skills of: - Disassemble every mechanism of 12.7mm gun - Identify defects of parts - Repair the Barrel - Repair the Receiver - Repair the Recoiling Mechanism - Repair the Feeding Mechanism - Repair Trigger Mechanism - Repair Back Plate Mechanism - Repair Cartridge box with belt - Assemble every part of the gun - perform Zeroing |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Repair of Cannons |
| Unit Code | IND SHA3 05 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding servicing, maintenance as well repairing different parts and mechanisms of cannons |

| Elements | Performance Criteria |
| --- | --- |
| 1. Repair Barrel | 1.1. Disassemble barrel and Inspect the degree and its specific problem. 1.2. Slight dents from the body of the barrel is removed using inappropriate tool. 1.3. Badly damaged barrel is rejected. 1.4. Scratches, dents, bulges from the chamber is removed using inappropriate tool. 1.5. Bended and damaged handle is adjusted. 1.6. Scratches, dents, bulge from lugs. And locks are removed using slight filing. 1.7. Badly damaged light eliminator is replaced and slight damage is maintained 1.8. Assemble and check its functionality. |
| 2.Repair Receiver | 2.1. Inspect receiver and identify its problems 2.2. Bend sears are adjusted, damaged sears are replaced. 2.3. Dents, scratches, bulges on the guide ways are removed using sand paper and file. 2.4. Remove old lubricant and replace with the new and appropriate lubricant 2.5. Finally check its functionality. |
| 3.Repair Breech block | 3.1.Remove allowable dents, nicks, scratches by filing 3.2. Damaged shaped recess, slots, guides are reshaped by filing. 3.3. Fill worn out striker assembly 3.4 Replace damaged striker/hammer 3.5 Replace damaged firing pin and firing spring. 3.6 Remove dents, nicks, and scratches from extractor and the housing. 3.7. Replace damaged extractor and spring. 3.8. Worn out bases are filed. 3.9. Broken or torn bases are replaced. 3.10. Remove dents nicks scratches from cocking cam /breech block ,shaped recess and lugs 3.11. Replace worn out pins, cocking cam, breech block, shaped recess and lugs. |
| 4. Repair Feed mechanism | .4.1. Damaged shutters are hammered. 4.2. Broken shutters are welded. 4.3. Worn out shutters are replaced. 4.4. Malfunctioning shutters are replaced. 4.5.Damaged pawl with spring and lever are filed 4.6. Broken pawl is maintained by welding. 4.7. Badly damaged pawl and feed lever are replaced. 4.8. Damaged carrier block slots, guide cleaned and filed 4.9. Dents, nicks, scratches are removed piston rods And piston head. 4.10. Broken piston rod and piston head are replaced 4.11. Damaged link ejectors are hammered. 4.12. Broken or torn ejectors are replaced. 4.13. Damaged pins and springs are replaced 4.14. Damaged bases are hammered. 4.15 Worn out bases are filed. 4.16. Broken or torn bases are replaced. 4.17. Damaged pins and springs are replaced. |
| 5.Repair Trigger mechanism and inter lock gear | 5.1. Check and inspect the defects of the trigger mechanism and inter-lock gear. 5.2. Removed allowable dents, nicks, scratches by filing. 5.3. Check and inspect trigger pin extrusion as per specifications 5.4. Fill trigger parts worn out part. 5.5. Replace damaged trigger spring. 5.6. Remove dents, nicks scratches from pin housing. 5.7. Replace damaged trigger base. |
| 6.Repair Back plate | 6.1. Disassemble back plate mechanism and Correct information is accessed and interpreted from Manufacturer’s and designer’s specifications. 6.2. Damaged and broken spring is replaced 6.3. Slight damaged teeth and key of the plate is adjusted using an appropriate tool 6.4 Badly damaged back plates are replaced. 6.5. Assembled and check malfunction of the back plate mechanism |
| 7.Repair Retraction mechanism | 7.1.Damaged and broken spring is replaced 7.2.Damaged wired cables are replaced 7.3.Damaged or bended flat springs are adjusted 7.4. Brocken flat springs are replaced 7.5.Brocken retracting handles are welded 7.6. Damaged rollers are replaced 7.8.Damaged guided ways are filed |
| 8.Repair Recoil buffer | 8.1. Disassemble recoil buffer and Correct information is accessed and interpreted from Manufacturer’s and designer’s specifications. 8.2. Quality of springs is checked and tested according to the standard working procedures. 8.3 Broken or weak springs are replaced correctly on its right position according to manufacturer’s specifications. 8.4. Assembled and check malfunction of the recoil mechanism |
| 9. Repair Cradle | - Cradle is checked and inspected based on the standard. - Minor dents on the cradle are permitted provided that they don’t hinder free travel of the recoil units, operations of the mechanisms as well as assembly and disassembly but if it is more is should maintained by welding and painted. - Trunnion holder with cracks is replaced by proper one. A crake in the cradle frame is welded at a length not in excess |
| 10.Repair Top carriage (swivel) | - Top Carriage is checked and inspected based on the standard - Slight dents from the body of the Top Carriage are removed using inappropriate tool. - Badly damaged Top Carriage is rejected. |
| 11. Repair Elevating mechanism | - Teeth are cleaned to standard to minimize effort applied - Bend hand wheel and lever is adjusted to the specification following standard work place procedures. - Broken hand wheel and lever is maintained according to work requirement and work place guidelines |
| 12. Repair Traversing mechanism | - Seriously damaged handle and wheel is replaced as per work place guidelines. - Damaged key is maintained to attain its standard size and dimension - Hardly damaged key is replaced by new one as per work procedures and guidelines |
| 13. Repair cartilage Box holder | 13.1. Disassemble cartilage box or belt and Correct information is accessed and interpreted from Manufacturer’s and designer’s specifications. 13.2. Slight deformation on the cartridge box is hammered. 13.3. Damaged chain of the belt is replaced. 13.4. Bend belt lug is adjusted. 13.5. Damaged pins of the belt are replaced. 13.6. Badly damaged cartridge box and belt is replaced. 13.7. Assembled and check malfunction of the cartridge box and belt. |
| 14.wheel bogies | 14.1Wheel bogies is checked and inspected based on the standard 14.2. Damaged wheel is replaced 14.3. Damaged nuts and valve pins are replaced |

| Variables | # Range |
| --- | --- |
| Barrel | May include but not limited to: - barrel of the 23mm Antiaircraft twin gun mount |
| Receiver | May include but not limited to: - receiver of the 23mm Antiaircraft twin gun mount |
| Trigger mechanism and interlock mechanism | May include but not limited to: - trigger mechanism and interlock mechanism of the 23mm Antiaircraft twin gun mount |
| Recoil buffer | May include but not limited to: - Recoil buffer of the 23mm Antiaircraft twin gun mount |
| Cradle | May include but not limited to: - Cradle of the 23mm Antiaircraft twin gun mount |
| Recoil mechanism | May include but not limited to: - recoil mechanism of the 23mm Antiaircraft twin gun mount |
| Occupational Health & Safety (OHS) | May include but not limited to: - Use of relevant Uniforms and protective when necessary - Use of Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones etc, - Maintain fire extinguishers and protection tools at appropriate place - Follow Occupational health and safety procedures designated for the task - Check and fulfill required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: - SPTA - Lifting machines - Scraper - Welding machine - Files - Belching chemicals(anti rust) - Manufacturing machines - Hammer, wire brush etc - Measuring tools |
| Types and Sources of Information | May include but not limited to: - written and graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate must demonstrate as follows: - Described procedures and precautions during bottom carriage with trail maintenance - Identified and inspected material for repair - Demonstrated safe working procedures - Designed and manufactured required spare parts - Inspected torsion bars, replaced broken and weak springs, and torsion bars - Measured the pin and holes on tie rod, diameter of the holes in the bracket and the pin, the hole of bushing and diameter of the journals of front bushing - Manufactured and assembled lock, dowel, nut and handle - Welded up cracks in the trails and spades - Stopped holes in the trail frame - Clean up work area and maintain equipment |
| Underpinning knowledge and attitude | Demonstrates Knowledge of: - Tactical behavior of the Anti aircraft machine gun - Ballistics characteristics of ZU-23 AAM gun. - Theory of the Fire control system - The functioning of the different mechanisms |
| Underpinning skills | Demonstrates Skills to: - Basic manufacturing - Basic designing and specification - General knowledge of maintenance - Work ethics - Basic Metallurgy - Technical drawing - Fundamentals small arms - Fundamentals of optics - Basic mathematics knowledge - Basic writing and speaking knowledge - General knowledge of material management and shop - Disassemble every mechanism of the AAMG - Identify malfunctioning parts - Manufacture defective manual using the data from the repair manual - Repair Barrel - Repair Receiver - Repair Breech block - Repair Feed mechanism - Repair Trigger mechanism and inter lock gear - Repair Back plate - Repair Re-tracking mechanism - Repair Recoil buffer - Repair Cradle - Repair Top carriage (swivel) - Repair Elevating mechanism - Repair Traversing mechanism - Assembling and checking the gun |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |  |
| --- | --- | --- |
| Unit Title |  | Set up Pneumatic and Hydraulic Systems |
| Unit Code |  | IND SHA3 06 0616 |
| Unit Descriptor | This unit covers the competence required to carry out the inspection, servicing and repairing of hydraulic and pneumatic systems. The unit includes identification and confirmation of work requirement, preparation for work, testing of systems, analysis of results, servicing and repairing of hydraulic and pneumatic systems and completion of work finalisation processes, including clean-up and documentation |  |

| Elements | Performance Criteria |
| --- | --- |
| 1. Prepare to undertake testing and servicing of hydraulic systems | 1.1 Nature and scope of work requirements are identified and Confirmed. 1.2 OHS requirements, including individual State/Territory regulatory requirements and personal protection needs are observed throughout the work. 1.3 Procedures and information such as workshop manuals and Specifications, and tooling required, are sourced. 1.4 Method options are analysed and those most appropriate to the circumstances are selected and prepared. 1.5 Technical requirements for testing and servicing of hydraulic systems are sourced and support equipment is identified and prepared. 1.6 Warnings in relation to working with hydraulics are observed. |
| 2. Test hydraulic systems and analyse results | 2.1 Methods for the system tests and implemented in accordance with workplace procedures and manufacturer specifications. 2.2 Results are compared with manufacturer specifications to indicate compliance or non-compliance. 2.3 Results are documented with evidence and supporting information and recommendation(s) made. 2.4 Report is processed in accordance with workplace procedures. |
| 3 . Conduct inspection and analyse results | 3.1. Methods for the inspection are implemented in accordance with workplace procedures and manufacturer specifications. 3.2. Inspection results are compared with manufacturer specifications to indicate compliance or non-compliance. 3.3. Results are documented with evidence and supporting information and recommendation(s) made. 3.4. Report is processed in accordance with workplace procedures. |
| - Carry out service and repair | 4.1. Methods for the service and repair are implemented in accordance with workplace procedures and manufacturer specifications. 5.2. Adjustments made during the service and repair are in accordance with manufacturer specifications. |

| Variable | Range |
| --- | --- |
| OHS requirements, | May include but not limited to: - Use of relevant Uniforms and protective when necessary - Use of Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones etc, - Maintain fire extinguishers and protection tools at appropriate place - Follow Occupational health and safety procedures designated for the task - Check and fulfill required safety devices before starting operation |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate must demonstrate as follows - Described procedures and precautions during operating pneumatic and hydraulic systems - Identified and inspected material for repair - Demonstrated safe working procedures - Designed and manufactured required spare parts - Inspected control valves, pipelines, safety valves, cylinder and actuating rod, replace broken and weak springs. - Manufactured and assembled valves, lines, cylinders, rods are inspected - Repair leakages in the systems - Clean up work area and maintain equipment |
| Underpinning knowledge and Attitudes | Demonstrates knowledge of: - OHS and environmental regulations/requirements, equipment, material and personal safety requirements - dangers of working with pneumatic systems - able to read and interpret schematic diagrams relevant to pneumatic systems - identification of application, purpose and operating principles - identification of pneumatic system schematic symbols |
| Underpinning skills | Demonstrates skills of: - reading, interpreting and following information on written job instructions, specifications, standard operating procedures, manufacturer instructions, charts/lists and drawings - checking and clarifying task-related information - following verbal instructions - orally reporting routine information - selecting appropriate processes for heating/quenching, tempering, annealing - setting up, adjusting and operating equipment |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Maintenance Level III |  |
| --- | --- |
| Unit Title | Repair of 122mm D-30 Howitzer |
| Unit Code | IND SHA3 07 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding servicing, maintenance as well repairing different parts and mechanisms of 122mm D-30 Howitzer. |

| Elements | Performance Criteria |
| --- | --- |
| 1.Prepar for work | - Work instructions are followed to determine job requirements - Job specifications are read and interpreted - Material for repair are identified and inspected for quality |
| - Maintain Barrel, Breech Mechanism with semiautomatic Gear, Guard Assembly, Trigger Mechanism | - Disassemble barrel and inspect the degree and its specific problem. - Slight dents from the body of the barrel is removed using inappropriate tool - Badly damaged barrel is rejected. - Scratches, dents, bulges from the chamber is removed using inappropriate tool - Remove allowable dents, nicks, scratches by filing - Damaged shaped recess, slots, guides are reshaped by filing. - Replace damaged firing pin and firing spring. - Remove dents, nicks, and scratches from extractor. - Replace damaged extractor and spring. - Remove dents nicks scratches from cocking cam /breech block ,shaped recess and lugs - Replace worn out pins, cocking cam, breech block, Shaped recess and lugs. - Damaged and worn out gears is maintained following work place procedure according to specification requirement. - Seriously damaged gears are replaced following work place procedure and work requirement. - Check and inspect the defects of the trigger mechanism Badly damaged trigger is rejected - Check and inspect the defects of the Guard Assembly. Badly damaged Guard Assembly is rejected |
| - Maintain Cradle and Recoil Mechanism | - Cradle is checked and inspected based on the standard. - Minor dents on the cradle are permitted provided that they don’t hinder free travel of the recoil units, operations of the mechanisms as well as assembly and disassembly but if it is more is should maintained by welding and painted. - Trunnion holder with cracks is replaced by proper one. - A crake in the cradle frame is welded at a length not in excess of 1/10 of frame section perimeter, having drilled holes, 3-6mm in die. At the end of cracks. - Cracks exceeds the permissible limit are welded and cover plates reinforced - Holes are stopped in the cradle frame by welding cover plates. - Plates are selected depending on the location of cracks and holes according to manufacturer’s and workplace specification limit - Lock defect is inspected following work place procedures. - Deformation and bulging on the lock is slightly machined and removed - Broken and weak spring is replaced according to its standard - Seriously damaged lock is replaced by new spare as per workplace procedures and guideline - Bronze structure is inspect following work place procedures and specification requirements - Seriously damaged bronze and worn out rubber are replaced by another one following work place procedures and according to and the specification - Parts to lubricated are lubricated using appropriate lubricant to reduce friction according to service manual specification - Rod gland is tightened up to the standard. - Gland packing is added to avoid leakage according work requirement and guidelines. - Nut is changed by another one according to job requirement and standard work procedure - Damaged bevel gear is maintained and worn out shems is replaced - Fluid leakage is checked to avoid accident or damage as per standard work requirement and guidelines. - Hardly damaged packing ring is replaced by new on as per standard work procedures and guidelines - Leakage through joint valve is checked through accepted procedures for safety. - Nut teeth are maintained by removing rust and scraps to the required standard. - Hardly damaged nozzle is replaced as per standard work procedures and service manual/guidelines. - Totally damaged valve is replaced by another one as per standard work procedures and service manual/guidelines. - Outer diameter of rod jacket is measured and compared with the standard. - Bend rods are adjusted using appropriate devices to assure perfect movement inside cylinder. - Damaged rods are chrome plated to achieve standard smoothness. - Worn-out piston jackets are replaced as per standard work procedures and service manual/guidelines. - Slightly deformed cylinders are adjusted using proper working devices to acquire its standard cylinders. - Rust is removed and external surface is painted to the standard - Seriously damaged cylinder is replaced with proper one following service manual/guidelines. |
| 3. Maintain Top Carriage, elevating and traversing mechanism, Equilibrator | - Top Carriage is checked and inspected based on the standard - Slight dents from the body of the Top Carriage are removed using inappropriate tool. - Badly damaged Top Carriage part/mechanism is rejected. - Teeth are cleaned to standard to minimize effort applied - Bend hand wheel and lever is adjusted to the specification following standard work place procedures. - Broken hand wheel and lever is maintained according to work requirement and work place guidelines - Seriously damaged handle and wheel is replaced as per work place guidelines. - Damaged key is maintained to attain its standard size and dimension - Hardly damaged key is replaced by new one as per work procedures and guidelines.. - Holes/Grove of key are measured and maintained using proper devices to fit with key as per work procedures and guidelines.. - Worn out gasket/shims are replaced following work procedures and guidelines. - Seriously damaged bronze and disks is replaced by another one as per work procedures and guidelines.. - Type of the bearing that fit to the requirement is identified based on catalogue specification - Damaged bearing is replaced following work procedures and guidelines to minimize friction. - Gears are checked and inspected according to work procedures and guidelines. - Worn out & undersized spacers are replaced according to its standard. - Scratches, dents, bulges and rust is removed to reduce friction - Slight pitch angle is maintained according to its standard. - Backlash is adjusted following standard work procedures. - Broken/worn out gears and snap rings are replaced as per requirement of the work - Maintain defective gear housing following standard work - Seriously damaged clutch is replaced following work place guidelines. - Seriously damaged connector and weak spring is replaced as per job requirement and work place guideline |
| 4. Maintain Wheeled Carriage, Wheels, Wheel Brake, Wheel Lifting Mechanism and Shield Assembly | - Specific problem tyre & wheel system is inspected with the proper mechanism. - Damaged washers and bearings are replaced according to the manufacturer’s service manual and workplace guidelines - Gland and spacer rings are inspected to verify its quality. - Worn out gland is replaced by proper one according to the manufacturer’s service manual and workplace guidelines - Damaged spacer rings are replaced by new one according to the manufacturer’s service manual and workplace guidelines - Broken ear is withdrawn in accordance with workplace guidelines - New ear is manufactured and welded it to the hub using the technical manual and following the OH & S instructions. - Allayer of metal is built up on the hub when the ear fracture makes it impossible to prepare the hub for installation of the ear using electrode in accordance with the manufacturer’s specification |
| 5 Maintain Bottom Carriage, Trails, Hydraulic Jack ,and Electrical Equipment | - Turn-off handle is welded to the rod following occupational health and safety(OHS) procedures using standard electrode - Damaged bevel gear is maintained and worn out shams is replaced - Specific problems on cone of valve, spring and collar are checked and identified following standard work place procedures. - Damaged valves and collars, broken and weak springs are replaced. - check and inspected the type of problem - Dents and through holes are maintained according to service manual. - The tolerable dimensions of through hole to be stopped with cover plates are to be not more than 35*50mm. - Damaged and worn out gears is maintained following work place procedure according to specification requirement. - Seriously damaged gears are replaced following work place procedure and work requirement. - Storage and different batteries are checked and maintained in accordance with specification. - Main switch is checked against the standard. |

| Variables | # Range |
| --- | --- |
| Barrel | May include but not limited to: - Barrel of 122mm howitzer |
| Cradle | May include but not limited to: - Cradle of 122mm howitzer |
| Top Carriage | May include but not limited to: - Top Carriage of 122mm howitzer |
| Occupational Health & Safety (OHS) | May include but not limited to: - Using relevant Uniforms and protective when necessary - Using Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones etc, - Maintaining fire extinguishers and protection tools at appropriate place - Following Occupational health and safety procedures designated for the task - Checking and fulfilling required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: - SPTA - Lifting machines - Scraper - Welding machine - Files - Belching chemicals(anti rust) - Manufacturing machines - Hammer, wire brush etc - Measuring tools |
| Types and Sources of Information | May include but not limited to: - written and graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate must demonstrate as follows: - Identified and inspected material for repair - Described principles and procedures for Barrel, Breech Mechanism ,Recoil Mechanism, elevating and traversing mechanism, Equilibrator, Wheels, Wheel Brake, Wheel Lifting ,jack mechanism maintenance - Demonstrated safe working procedures - Replaced damaged valves and collars, broken and weak springs. - maintained bevel gear - Cleaned up work area and maintained equipment |
| Underpinning knowledge and attitude | Demonstrate Knowledge of: - Basic manufacturing - Basic designing and specification - General knowledge of maintenance - Work ethics - Basic Metallurgy - Technical drawing - Fundamentals of artillery - Fundamentals of optics - Basic mathematics knowledge - Basic writing and speaking knowledge - General knowledge of material management and shop - Functioning of heavy arms - Tactical employment of heavy arms - Ballistics characteristics of 122mm howitzer - Usage of different types of ammunitions |
| Underpinning skill | Demonstrates Skills to: - Disassemble every mechanism of the gun - Identify troubles on each mechanism - Check the recuperator and recoil break - Fill oil and air to the recoil mechanism - Maintain traversing and elevating mechanism - Maintain equilibrator - Assemble and check the gun mechanisms - Boar sighting |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Maintenance Level III |  |
| --- | --- |
| Unit Title | Repair of 130mm M-46 Field Gun |
| Unit Code | IND SHA3 08 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding servicing, maintenance as well repairing different parts and mechanisms of 130mm M46 Field Gun. |

| Elements | Performance Criteria |
| --- | --- |
| 1.Prepar for work | - Work instructions are followed to determine job requirement - Job specifications are read and interpreted - Material for repair are identified and inspected for quality |
| 2. Maintain Barrel | - Disassemble barrel and inspect the degree and its specific problem. - Slight dents from the body of the barrel is removed using inappropriate tool - Badly damaged barrel is rejected. - Scratches, dents, bulges from the chamber is removed using inappropriate tool |
| 3. Maintain Breech mechanism | - Scratches, dents, bulges from the chamber is removed using inappropriate tool - Remove allowable dents, nicks, scratches by filing - Damaged shaped recess, slots, guides are reshaped by filing. - Replace damaged firing pin and firing spring. - Remove dents, nicks, and scratches from extractor. Replace damaged extractor and spring. - Remove dents nicks scratches from cocking cam /breech block ,shaped recess and lugs - Replace worn out pins, cocking cam ,breech block, Shaped recess and lugs. |
| 4. Maintain Cradle | 4.1. Cradle is checked and inspected based on the standard. - Minor dents on the cradle are permitted provided that they don’t hinder free travel of the recoil units, operations of the mechanisms as well as assembly and disassembly but if it is more is should maintained by welding and painted. - Trunnion holder with cracks is replaced by proper one. - A crake in the cradle frame is welded at a length not in excess of 1/10 of frame section perimeter, having drilled holes, 3-6mm in die. At the end of cracks. - Cracks exceeds the permissible limit are welded and cover plates reinforced - Holes are stopped in the cradle frame by welding cover plates. - Plates are selected depending on the location of cracks and holes according to manufacturer’s and workplace specification limit |
| - Maintain Recoil system | - Parts to lubricated are lubricated using appropriate lubricant to reduce friction according to service manual specification - Rod gland is tightened up to the standard. - Gland packing is added to avoid leakage according to work requirement and guidelines. - Nut is changed by another one according to job requirement and standard work procedure - Damaged bevel gear is maintained and worn out shems is replaced - Fluid leakage is checked to avoid accident or damage as per standard work requirement and guidelines. - Hardly damaged packing ring is replaced by new on as per standard work procedures and guidelines - Leakage through joint valve is checked through accepted procedures for safety. - Nut teeth are maintained by removing rust and scraps to the required standard. - Hardly damaged nozzle is replaced as per standard work procedures and service manual/guidelines.. - Totally damaged valve is replaced by another one as per standard work procedures and service manual/guidelines. - Outer diameter of rod jacket is measured and compared with the standard. - Bend rods are adjusted using appropriate devices to assure perfect movement inside cylinder. - Damaged rods are chrome plated to achieve standard smoothness. - Worn-out piston jackets are replaced as per standard work procedures and service manual/guidelines. - Slightly deformed cylinders are adjusted using proper working devices to acquire its standard cylinders. - Rust is removed and external surface is painted to the standard - Seriously damaged cylinder is replaced with proper one following service manual/guidelines. |
| 6. Maintain Top carriage and Shield assembly | - Top Carriage and Shield assembly e is checked and inspected based on the standard - Slight dents from the body of the Top Carriage and Shield assembly are removed using inappropriate tool. - Badly damaged Top Carriage and Shield assembly is rejected. - Teeth are cleaned to standard to minimize effort applied |
| - Maintain Elevating mechanism | - Teeth are cleaned to standard to minimize effort applied - Bend hand wheel and lever is adjusted to the specification following standard work place procedures. - Broken hand wheel and lever is maintained according to work requirement and work place guidelines - Seriously damaged handle and wheel is replaced as per work place guidelines. - Damaged key is maintained to attain its standard size and dimension - Hardly damaged key is replaced by new one as per work procedures and guidelines.. - Holes/Grove of key are measured and maintained using proper devices to fit with key as per work procedures and guidelines.. - Worn out gasket/shims are replaced following work procedures and guidelines. - Seriously damaged bronze and disks is replaced by another one as per work procedures and guidelines.. |
| 8 Maintain Traversing mechanism | - Damaged bearing is replaced following work procedures and guidelines to minimize friction. - Gears are checked and inspected according to work procedures and guidelines. - Worn out & undersized spacers are replaced according to its standard. - Scratches, dents, bulges and rust is removed to reduce friction - Slight pitch angle is maintained according to its standard. - Backlash is adjusted following standard work procedures. - Broken/worn out gears and snap rings are replaced as per requirement of the work - Maintain defective gear housing following standard work - Seriously damaged clutch is replaced following work place guidelines. |
| - Maintain Equilibrator | 9.1. Maintain defective gear housing following standard work 9.2. Seriously damaged clutch is replaced following work place guidelines 9.3. Seriously damaged connector and weak spring is replaced as per job requirement and work place guideline |
| 10 Maintain Bottom Carriage and Trails | - Disassemble Bottom Carriage and Trails and inspect the degree and its specific problem. - A slight dent from the body of the Bottom Carriage and Trails is removed using inappropriate tool. - Badly damaged Bottom Carriage and Trails is rejected. - Turn-off handle is welded to the rod following occupational health and safety(OHS) procedures using standard electrode - Damaged bevel gear is maintained and worn out shams is replaced - Specific problems on cone of valve, spring and collar are checked and identified following standard work place procedures. - Damaged valves and collars, broken and weak springs are replaced. - check and inspected the type of problem - Dents and through holes are maintained according to service manual. |
| - Maintain Winch and Jacks | - Worn out gasket/shims are replaced following work procedures and guidelines. - Broken/worn out gears and snap rings are replaced as per requirement of the work |
| - 12. Maintain Limber and Wheels and Wheel brake | - check and inspected the type of problem - broken and weak springs are replaced. - Badly damaged Wheels are replaced - Badly damaged Limber is rejected |

| Variables | # Range |
| --- | --- |
| Barrel | May include but not limited to: - Barrel of 130mm howitzer |
| Cradle | May include but not limited to: - Cradle of 130mm howitzer |
| Top Carriage | May include but not limited to: - Top Carriage of 130mm howitzer |
| Occupational Health & Safety (OHS) | May include but not limited to: - Using relevant Uniforms and protective when necessary - Using Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones etc, - Maintaining fire extinguishers and protection tools at appropriate place - Following Occupational health and safety procedures designated for the task - Checking and fulfilling required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: - SPTA - Lifting machines - Scraper - Welding machine - Files - Bleaching chemicals(anti rust) - Manufacturing machines - Hammer, wire brush etc - Measuring tools |
| Types and Sources of Information | Information source may include variable written and graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate must demonstrate as follows: - Identified and inspected material for repair - Described principles and procedures for jack mechanism maintenance - Demonstrated safe working procedures - Replaced damaged valves and collars, broken and weak springs, worn out shems - maintained Traversing mechanism. - Cleaned up work area and maintained equipment |
| Underpinning knowledge and attitude | Demonstrate Knowledge of: - Basic manufacturing - Basic designing and specification - General knowledge of maintenance - Work ethics - Basic Metallurgy - Technical drawing - Fundamentals of artillery - Fundamentals of optics - Basic mathematics knowledge - Basic writing and speaking knowledge - General knowledge of material management and shop - Functioning of heavy arms - Tactical employment of 130mm artillery - Ballistics characteristics of 130mm howitzer - Usage of different types of ammunitions |
| Underpinning skill | Demonstrates Skills to: - Disassemble every mechanism of the gun - Identify troubles on each mechanism - Check the recuperator and recoil break - Fill oil and air to the recoil mechanism - Maintain traversing and elevating mechanism - Maintain equilibrator - Assemble and check the gun mechanisms - Boar sighting |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Repair of Tank Guns |
| Unit Code | IND SHA3 09 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding servicing, maintenance as well repairing different parts and mechanisms of Tank Gun. |

| Elements | Performance Criteria |
| --- | --- |
| 1.Prepar for work | - Work instructions are followed to determine job requirements - Job specifications are read and interpreted - Material for repair are identified and inspected for quality |
| 2. Maintain Barrel | - Disassemble barrel and inspect the degree and its specific problem. - Slight dents from the body of the barrel is removed using inappropriate tool - Badly damaged barrel is rejected. - Scratches, dents, bulges from the chamber is removed using inappropriate tool |
| 3. Maintain Breech mechanism | 3.1. Scratches, dents, bulges from the chamber is removed using inappropriate tool 3.2. Remove allowable dents, nicks, scratches by filing 3.3. Damaged shaped recess, slots, guides are reshaped by filing. - Replace damaged firing pin and firing spring. - Remove dents, nicks, and scratches from extractor. Replace damaged extractor and spring. - Remove dents nicks scratches from cocking cam /breech block ,shaped recess and lugs - Replace worn out pins, cocking cam ,breech block, Shaped recess and lugs. |
| 4. Maintain Cradle | - Cradle is checked and inspected based on the standard. - Minor dents on the cradle are permitted provided that they don’t hinder free travel of the recoil units, operations of the mechanisms as well as assembly and disassembly but if it is more is should maintained by welding and painted. - Trunnion holder with cracks is replaced by proper one. - A crake in the cradle frame is welded at a length not in excess of 1/10 of frame section perimeter, having drilled holes, 3-6mm in die. At the end of cracks. - Cracks exceeds the permissible limit are welded and cover plates reinforced - Holes are stopped in the cradle frame by welding cover plates. - Plates are selected depending on the location of cracks and holes according to manufacturer’s and workplace specification limit |
| 5. Maintain Recoil system | - Parts to lubricated are lubricated using appropriate lubricant to reduce friction according to service manual specification - Rod gland is tightened up to the standard. - Gland packing is added to avoid leakage according to work requirement and guidelines. - Nut is changed by another one according to job requirement and standard work procedure - Damaged bevel gear is maintained and worn out shems is replaced - Fluid leakage is checked to avoid accident or damage as per standard work requirement and guidelines. - Hardly damaged packing ring is replaced by new on as per standard work procedures and guidelines - Leakage through joint valve is checked through accepted procedures for safety. - Nut teeth are maintained by removing rust and scraps to the required standard. - Hardly damaged nozzle is replaced as per standard work procedures and service manual/guidelines. - Totally damaged valve is replaced by another one as per standard work procedures and service manual/guidelines. - Outer diameter of rod jacket is measured and compared with the standard. - Bend rods are adjusted using appropriate devices to assure perfect movement inside cylinder. - Damaged rods are chrome plated to achieve standard smoothness. - Worn-out piston jackets are replaced as per standard work procedures and service manual/guidelines. - Slightly deformed cylinders are adjusted using proper working devices to acquire its standard cylinders. - Rust is removed and external surface is painted to the standard - Seriously damaged cylinder is replaced with proper one following service manual/guidelines. |
| 6. Maintain Shield assembly | - Shield assembly e is checked and inspected based on the standard - Slight dents from the body of the Top Carriage and Shield assembly are removed using inappropriate tool. - Badly damaged Shield assembly is rejected. - Teeth are cleaned to standard to minimize effort applied |
| - Maintain Elevating mechanism | - Teeth are cleaned to standard to minimize effort applied - Bend hand wheel and lever is adjusted to the specification following standard work place procedures. - Broken hand wheel and lever is maintained according to work requirement and work place guidelines - Seriously damaged handle and wheel is replaced as per work place guidelines. - Damaged key is maintained to attain its standard size and dimension - Hardly damaged key is replaced by new one as per work procedures and guidelines.. - Holes/Grove of key are measured and maintained using proper devices to fit with key as per work procedures and guidelines.. - Worn out gasket/shims are replaced following work procedures and guidelines. - Seriously damaged bronze and disks is replaced by another one as per work procedures and guidelines.. |
| 8 Maintain Traversing mechanism | - Damaged bearing is replaced following work procedures and guidelines to minimize friction. - Gears are checked and inspected according to work procedures and guidelines. - Worn out & undersized spacers are replaced according to its standard. - Scratches, dents, bulges and rust is removed to reduce friction - Slight pitch angle is maintained according to its standard. - Backlash is adjusted following standard work procedures. - Broken/worn out gears and snap rings are replaced as per requirement of the work - Maintain defective gear housing following standard work - Seriously damaged clutch is replaced following work place guidelines. |
| - Maintain Equilibrator | - Maintain defective gear housing following standard work - Seriously damaged clutch is replaced following work place guidelines. - Seriously damaged connector and weak spring is replaced as per job requirement and work place guideline |
| 10 Maintain Bottom Carriage Turret | - Disassemble Bottom Carriage and Turret and inspect the degree and its specific problem. - A slight dent from the body of the Bottom Carriage and Trails is removed using inappropriate tool. - Badly damaged Bottom Carriage and Trails is rejected. - Turn-off handle is welded to the rod following occupational health and safety(OHS) procedures using standard electrode - Damaged bevel gear is maintained and worn out shams is replaced - Specific problems on cone of valve, spring and collar are checked and identified following standard work place procedures. - Damaged valves and collars, broken and weak springs are replaced. - Check and inspect the type of problem - Dents and through holes are maintained according to service manual. |

| Variables | # Range |
| --- | --- |
| Material | May include but not limited to: - Mechanical tool set, electrical tool set and special tools |
| Cradle | May include but not limited to: - cradle of T-62 and T-72 tank |
| Barrel | May include but not limited to: - barrel of T-62 and T-72 tank |
| Bottom Carriage and turret | May include but not limited to: - Bottom carriage and turret of T-62 and T-72 tank |
| Occupational Health & Safety (OHS) | May include but not limited to: - Using relevant Uniforms and protective when necessary - Using Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones etc, - Maintaining fire extinguishers and protection tools at appropriate place - Following Occupational health and safety procedures designated for the task - Checking and fulfilling required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: - SPTA - Lifting machines - Scraper - Welding machine - Files - Belching chemicals(anti rust) - Manufacturing machines - Hammer, wire brush etc - Measuring tools |
| Types and Sources of Information | Information source may include variable written and graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate must demonstrate as follows: - Identified and inspected material for Barrel , Breech mechanism, Recoil system, Traversing mechanism, Elevating mechanism. - Described principles and procedures for maintenance - Demonstrated safe working procedures - Replaced damaged valves and collars, broken and weak springs, worn out shems - Cleaned up work area and maintained equipment |
| Underpinning knowledge and attitude | Demonstrates Knowledge of: - Basic manufacturing - Basic designing and specification - General knowledge of maintenance - Basic electricity - Electrical machine - Work ethics - Basic Metallurgy - Technical drawing - Fundamentals of artillery - Fundamentals of optics - Basic mathematics knowledge - Basic writing and speaking knowledge - General knowledge of material management and shop - The combat characteristics of T-62 and T-72 tanks - Ballistics characteristics of T-62 and T-72 tanks - Design considerations of the tank gun - Working principle of the recoiling parts - The fire control system - Mechanical and electrical mechanisms of the elevating and traversing mechanisms |
| Underpinning skill | Demonstrate Skills to: - Checking and inspecting the barrel bore for scratches, scores, cracks - Disassembling and troubleshooting the breech mechanism - Checking the percussion mechanism - Checking the recoil mechanism’s oil and gas and fill if necessary - Maintaining elevating and traversing mechanism. - Check the equilibrator for functionality - Manufacture some parts on machines using the specification on the repair manual |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Servicing of 155 APU Howitzer Gun |
| Unit Code | IND SHA3 10 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding servicing, as well maintenance different parts and mechanisms of 155 APU Howitzer gun |

| Elements | Performance Criteria |
| --- | --- |
| - Prepare for work | - Work instructions are followed to determine job requirements - Job specifications are read and interpreted - Material for repair are identified and inspected for quality |
| - Servicing firing part | - Disassemble barrel and inspect the degree and its specific problem. - Slight dents from the body of the barrel is removed using inappropriate tool - Badly damaged barrel is rejected. - Remove dents nicks scratches from cocking cam /breech block ,shaped recess and lugs - Replace worn out pins, cocking cam, breech block, shaped recess and lugs. - Fluid leakage is checked to avoid accident or damage as per standard work requirement and guidelines. - Hardly damaged packing ring is replaced by new on as per standard work procedures and guidelines |
| - Servicing Aiming part | - Teeth are cleaned to standard to minimize effort applied - Bend hand wheel and lever is adjusted to the specification following standard work place procedures. - Broken hand wheel and lever is maintained according to work requirement and work place guidelines - Seriously damaged handle and wheel is replaced as per work place guidelines. - Maintain defective gear housing following standard work - Seriously damaged clutch is replaced following work place guidelines. - Seriously damaged connector and weak spring is replaced as per job requirement and work place guideline |
| - Servicing Carriage part | - Cradle is checked and inspected based on the standard. - Minor dents on the cradle are permitted provided that they don’t hinder free travel of the recoil units, operations of the mechanisms as well as assembly and disassembly but if it is more is should maintained by welding and painted. - Trunnion holder with cracks is replaced by proper one. - A crake in the cradle frame is welded at a length not in excess of 1/10 of frame section perimeter, having drilled holes, 3-6mm in die. At the end of cracks. - Cracks exceeds the permissible limit are welded and cover plates reinforced - Holes are stopped in the cradle frame by welding cover plates. - Plates are selected depending on the location of cracks and holes according to manufacturer’s and workplace specification limit |
| - Servicing traveling part | - Parts to lubricated are lubricated using appropriate lubricant to reduce friction according to service manual specification - Rod gland is tightened up to the standard. - Gland packing is added to avoid leakage according to work requirement and guidelines. - Nut is changed by another one according to job requirement and standard work procedure - Damaged bevel gear is maintained and worn out shems is replaced - Fluid leakage is checked to avoid accident or damage as per standard work requirement and guidelines. - Hardly damaged packing ring is replaced by new on as per standard work procedures and guidelines - Leakage through joint valve is checked through accepted procedures for safety. - Nut teeth are maintained by removing rust and scraps to the required standard. - Hardly damaged nozzle is replaced as per standard work procedures and service manual/guidelines. - Totally damaged valve is replaced by another one as per standard work procedures and service manual/guidelines. - Outer diameter of rod jacket is measured and compared with the standard. - Bend rods are adjusted using appropriate devices to assure perfect movement inside cylinder. - Damaged rods are chrome plated to achieve standard smoothness. - Worn-out piston jackets are replaced as per standard work procedures and service manual/guidelines. - Slightly deformed cylinders are adjusted using proper working devices to acquire its standard cylinders. - Rust is removed and external surface is painted to the standard Seriously damaged cylinder is replaced with proper one following service manual/guidelines. |
| - Servicing auxiliary propelling unit | - Top Carriage and Shield assembly is checked and inspected based on the standard - Slight dents from the body of the Top Carriage and Shield assembly are removed using inappropriate tool. - Badly damaged Top Carriage and Shield assembly is rejected. - Teeth are cleaned to standard to minimize effort applied |

| Variables | # Range |
| --- | --- |
| Barrel | May include but not limited to: - Barrel of 155 APU Howitzer |
| Cradle | May include but not limited to: - Cradle of 155 APU Howitzer |
| Occupational Health & Safety (OHS) | May include but not limited to: - Using relevant Uniforms and protective when necessary - Using Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones etc, - Maintaining fire extinguishers and protection tools at appropriate place - Following Occupational health and safety procedures designated for the task - Checking and fulfilling required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: - SPTA - Lifting machines - Scraper - Welding machine - Files - Belching chemicals(anti rust) - Manufacturing machines - Hammer, wire brush etc - Measuring tools |
| Types and Sources of Information | May include but not limited to: - written and graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate must demonstrate as follows: - Identified and inspected material for repair - Described principles and procedures for Servicing firing part, Servicing Aiming part, Servicing Carriage part, servicing traveling part and Servicing auxiliary propelling unit of 155mm APU howitzer. - Demonstrated safe working procedures - Replaced damaged valves and collars, broken and weak springs, worn out shems - Cleaned up work area and maintained equipment |
| Underpinning Knowledge and Attitude | Demonstrate: - Basic manufacturing - Basic designing and specification - General knowledge of maintenance - Basic electricity - Electrical machine - Work ethics - Basic Metallurgy - Technical drawing - Fundamentals of artillery - Fundamentals of optics - Basic mathematics knowledge - Basic writing and speaking knowledge - General knowledge of material management and shop - The speciality of 155 APU Howitzer - The combat characteristics of 155 APU Howitzer - The ballistics of 155 APU Howitzer |
| Underpinning skill | - Inspect all the mechanisms - Identify troubles - Servicing firing part - Servicing Aiming part - Servicing Carriage part - Servicing traveling part - Servicing. auxiliary propelling unit |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Servicing of 152 mm2C19 Howitzer |
| Unit Code | IND SHA3 11 0616 |
| Unit Descriptor | This unit involves accomplishment of activities regarding mainly servicing in the same cases maintenance different parts and mechanisms of 152mm 2c19 Howitzer. |

| Elements | Performance Criteria |
| --- | --- |
| - prepare for work | - Work instructions are followed to determine job requirements - Job specifications are read and interpreted - Material for repair are identified and inspected for quality |
| - Service and Maintain Barrel | - Disassemble barrel and inspect the degree and its specific problem. - Slight dents from the body of the barrel are removed using appropriate tool. - Badly damaged barrel is rejected. - Scratches, dents, bulges from the chamber are removed using inappropriate tool. |
| - Service and Maintain Breech mechanism | - A scratch, dents, bulges from the chamber is removed using inappropriate tool. - Remove allowable dents, nicks, scratches by filing. - Damaged shaped recess, slots, guides are reshaped by filing. - Replace damaged firing pin and firing spring. - Remove dents, nicks, and scratches from extractor. Replace damaged extractor and spring. - Remove dents nicks scratches from cocking cam /breech block, shaped recess and lugs. - Replace worn out pins, cocking cam, breech block, shaped recess and lugs. |
| - Service and Maintain equilibrator mechanism | - Equilibrator mechanism is checked and inspected based on the standard. - Plates are selected depending on the location of cracks and holes according to manufacturer’s and workplace specification limit. |
| - Service and Maintain Recoil system | - Parts to lubricated are lubricated using appropriate lubricant to reduce friction according to service manual specification. - Rod gland is tightened up to the standard. - Gland packing is added to avoid leakage according to work requirement and guidelines. - Nut is changed by another one according to job requirement and standard work procedure. - Damaged bevel gear is maintained and worn out shems is replaced. - Fluid leakage is checked to avoid accident or damage as per standard work requirement and guidelines. - Hardly damaged packing ring is replaced by new on as per standard work procedures and guidelines. - Leakage through joint valve is checked through accepted procedures for safety. - Nut teeth are maintained by removing rust and scraps to the required standard. - Hardly damaged nozzle is replaced as per standard work procedures and service manual/guidelines. |
| - Service and Maintain safety mechanism | - Safety mechanism checked and inspected based on the standard. - Damaged traversing locks are replaced - Lose electric cables are replaced - Change lever springs are replaced - Damaged firing buttons are replaced - Clean and inspect the chamber |
| - Service and Maintain Elevating mechanism | - Teeth are cleaned to standard to minimize effort applied. - Bend hand wheel and lever is adjusted to the specification following standard work place procedures. - Broken hand wheel and lever is maintained according to work requirement and work place guidelines. - Seriously damaged handle and wheel is replaced as per work place guidelines. - Damaged key is maintained to attain its standard size and dimension. - Hardly damaged key is replaced by new one as per work procedures and guidelines. - Holes/Grove of key are measured and maintained using proper devices to fit with key as per work procedures and guidelines. - Worn out gasket/shims are replaced following work procedures and guidelines. - Seriously damaged bronze and disks is replaced by another one as per work procedures and guidelines. |
| - Service and Maintain Traversing mechanism | - Damaged bearing is replaced following work procedures and guidelines to minimize friction. - Gears are checked and inspected according to work procedures and guidelines. - Worn out & undersized spacers are replaced according to its standard. - Scratches, dents, bulge and rust is removed to reduce friction. - Slight pitch angle is maintained according to its standard. - Backlash is adjusted following standard work procedures. - Broken/worn out gears and snap rings are replaced as per requirement of the work. - Maintain defective gear housing following standard work. - Seriously damaged clutch is replaced following work place guidelines. |
| - Service and Maintain Equilibrator | - Maintain defective gear housing following standard work - Seriously damaged clutch is replaced following work place guidelines. - Seriously damaged connector and weak spring is replaced as per job requirement and work place guideline. |
| - Service and Maintain electrical aiming 2346(stabilizer) | - Check and inspected the type of problem. - Dents and through holes are maintained according to service manual. - Service projectile (cartridge) storage rack. - Checked and inspect Hydraulic system of the 152mm 2C19 howitzer. - Service cartridge case loading mechanism. - Service barrel locker mechanism. - Service the electrical equipments of 152mm 2C19 howitzer. - Service General Advantage of loading parts of 6эµ19 and control equipments. |

| Variable | # Range |
| --- | --- |
| Barrel | May include but not limited to: - Barrel of the 152mm 2c Howitzer. |
| Equilibrator mechanism | May include but not limited to: - Equilibrator mechanism of the152mm 2c Howitzer |
| Occupational Health & Safety (OHS) | May include but not limited to: - Using relevant Uniforms and protective when necessary - Using Chemical prove gowns, rubber boots of appropriate size, Goggles, respirators, helmet, and head phones etc, - Maintaining fire extinguishers and protection tools at appropriate place - Following Occupational health and safety procedures designated for the task - Checking and fulfilling required safety devices before starting operation |
| Tools and Equipment | May include but not limited to: - SPTA - Lifting machines - Scraper - Welding machine - Files - Belching chemicals(anti rust) - Manufacturing machines - Hammer, wire brush etc - Measuring tools |
| Types and Sources of Information | May include but not limited to: - written and graphical instructions, work bulletins, data sheet, diagrams or sketches - Safety work procedures/manual - Workplace guidelines - Manufacturer’s diagrams, charts - Manufacturer’s catalogue/specification manual. - Manufacturer’s service and operation manuals - Design specification manual - Repair request documentation - Manufacturing and designing specifications and instructions - Records and reports - Internet - Virtual library |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Assessment requires evidence that the candidate: - Identified and inspected material for repair - Described principles and working procedures for mechanism - Service and minor Maintain Barrel, Breech mechanism , Cradle , Recoil system , Top carriage and Shield assembly , Elevating mechanism , Traversing mechanism , Equilibrator and Bottom Carriage - Demonstrated safe working procedures - Replaced damaged valves and collars, broken and weak springs, worn out shems - Cleaned up work area and maintained equipment |
| Underpinning knowledge and attitudes | - Basic manufacturing - Basic designing and specification - General knowledge of maintenance - Basic electricity - Electrical machine - Work ethics - Basic Metallurgy - Technical drawing - Fundamentals of artillery - Fundamentals of optics - Basic mathematics knowledge - Basic writing and speaking knowledge - General knowledge of material management and shop organization - Broad knowledge of input/output devices - Operating characteristics of 152mm 2c Howitzer - Combat characteristics 152mm 2c Howitzer - Ballistics of 152mm 2c Howitzer - Function of micro switches |
| Underpinning skills | - Service and Maintain Barrel - Service and Maintain Breech mechanism - Service and Maintain equilibrator mechanism - Service and Maintain Recoil system - Service and Maintain safety mechanism - Service and Maintain Elevating mechanism - Service and Maintain Traversing mechanism - Service and Maintain Equilibrator - Service and Maintain electrical aiming 2346(stabilizer) |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Monitor Implementation of Work Plan/Activities |
| Unit Code | IND SHA3 12 0616 |
| Unit Descriptor | This unit covers competence required to oversee and monitor the quality of work operations within an enterprise. This unit may be carried out by team leaders or supervisors. |

| Elements | Performance Criteria |
| --- | --- |
| - Monitor and improve workplace operations | - Efficiency and service levels are monitored on an ongoing basis. - Operations in the workplace have been supported overall enterprise goals and quality assurance initiatives. - Quality problems and issues are promptly identified and adjustments made accordingly. - Procedures and systems are changed in consultation with colleagues to improve efficiency and effectiveness. - Colleagues are consulted about ways to improve efficiency and service levels. |
| - Plan and organise workflow | - Current workload of colleagues is accurately assessed. - Work is scheduled in a manner which enhances efficiency and customer service quality. - Work is delegated to appropriate people in accordance with principles of delegation. - Workflow is assessed against agreed objectives and timelines and colleagues are assisted in prioritisation of workload. - Input regarding staffing needs is provided to appropriate management. |
| - Maintain workplace records | - Workplace records are accurately completed and submitted within required timeframes. - Where appropriate, completion of records is delegated and monitored prior to submission. |
| - Solve problems and make decisions | - Workplace problems are promptly identified and considered from an operational and customer service perspective. - Short term action is initiated to resolve the immediate problem where appropriate. - Problems are analysed for any long term impact and potential solutions assessed and actioned in consultation with relevant colleagues. - Where problem is raised by a team member, they are encouraged to participate in solving the problem. - Follow up action is taken to monitor the effectiveness of solutions in the workplace. |

| Variables | Range |
| --- | --- |
| Problems | May include but not limited to: - difficult customer service situations - equipment breakdown/technical failure - delays and time difficulties - competence |
| Workplace records | May include but is not limited to: - staff records and regular performance reports |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Demonstrates skills and knowledge in: - ability to effectively monitor and respond to a range of common operational and service issues in the workplace - understanding of the role of staff involved in workplace monitoring - knowledge of quality assurance, principles of workflow planning, delegation and problem solving |
| Underpinning Knowledge and Attitudes | Demonstrate knowledge of: - roles and responsibilities in monitoring work operations - overview of leadership and management responsibilities - principles of work planning and principles of delegation - typical work organization methods appropriate to the sector - quality assurance principles and time management - problem solving and decision making processes - industrial and/or legislative issues which affect short term work organization as appropriate to industry sector |
| Underpinning Skills | Demonstrate skills to: - monitor and improve workplace operations - plan and organize workflow - maintain workplace records |
| Resource Implications | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Apply Quality Control |
| Unit Code | IND SHA3 13 0616 |
| Unit Descriptor | This unit covers the knowledge, attitudes and skills required in applying quality control in the workplace. |

| Elements | Performance Criteria |
| --- | --- |
| - Implement quality standards | - Agreed quality standard and procedures are acquired and confirmed. - Standard procedures are introduced to organizational staff/personnel. - Quality standard and procedures documents are provided to employees in accordance with the organization policy. - Standard procedures are revised / updated when necessary. |
| - Assess quality of service delivered | - Services delivered are quality checked against organization quality standards and specifications. - Service delivered are evaluated using the appropriate evaluation quality parameters and in accordance with organization standards. - Causes of any identified faults are identified and corrective actions taken in accordance with organization policies and procedures. |
| - Record information | - Basic information on the quality performance is recorded in accordance with organization procedures. - Records of work quality are maintained according to the requirements of the organization. |
| - Study causes of quality deviations | - Causes of deviations from final outputs or services are investigated and reported in accordance with organization procedures. - Suitable preventive action is recommended based on organization quality standards and identified causes of deviation from specified quality standards of final service or output. |
| - Complete documentation | - Information on quality and other indicators of service performance is recorded. - All service processes and outcomes are recorded. |

| Variable | Range |
| --- | --- |
| Quality check | May include but not limited to: - Check against design / specifications - Visual inspection and Physical inspection |
| Quality standards | May include but not limited to: - Materials - Components - Process - Procedures |
| Quality parameters | May include but not limited to: - Standard Design / Specifications - Material Specification |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Demonstrates skills and knowledge to: - Check completed work continuously against organization standard - Identify and isolate faulty or poor service - Check service delivered against organization standards - Identify and apply corrective actions on the causes of identified faults or error - Record basic information regarding quality performance - Investigate causes of deviations of services against standard - Recommend suitable preventive actions |
| Underpinning Knowledge | Demonstrates knowledge of: - Relevant quality standards, policies and procedures - Characteristics of services - Safety environment aspects of service processes - Evaluation techniques and quality checking procedures - Workplace procedures and reporting procedures |
| Underpinning Skills | Demonstrates skills to: - interpret work instructions, specifications and standards appropriate to the required work or service - carry out relevant performance evaluation - maintain accurate work records - meet work specifications and requirements - communicate effectively within defined workplace procedures |
| Resource Implications | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Lead Workplace Communication |
| Unit Code | IND SHA3 14 0616 |
| Unit Descriptor | This unit covers the knowledge, attitudes and skills needed to lead in the dissemination and discussion of information and issues in the workplace. |

| Elements | Performance Criteria |
| --- | --- |
| 1. Communicate information about workplace processes | - Appropriate communication method is selected. - Multiple operations involving several topics areas are communicated accordingly. - Questions are used to gain extra information. - Correct sources of information are identified. - Information is selected and organized correctly. - Verbal and written reporting is undertaken when required. - Communication skills are maintained in all situations. |
| 2. Lead workplace discussion | - Response to workplace issues is sought. - Response to workplace issues are provided immediately. - Constructive contributions are made to workplace discussions on such issues as production, quality and safety. - Goals/objectives and action plan undertaken in the workplace are communicated. |
| 3. Identify and communicate issues arising in the workplace | - Issues and problems are identified as they arise. - Information regarding problems and issues are organized coherently to ensure clear and effective communication. - Dialogue is initiated with appropriate staff/personnel. - Communication problems and issues are raised as they arise. |

| Variable | Range |
| --- | --- |
| Methods of communication | May include but not limited to: - Non-verbal gestures - Verbal - Face to face - Two-way radio - Speaking to groups - Using telephone - Written - Using Internet - Cell phone |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Demonstrates skills and knowledge to: - Deal with a range of communication/information at one time - Make constructive contributions in workplace issues - Seek workplace issues effectively - Respond to workplace issues promptly - Present information clearly and effectively written form - Use appropriate sources of information - Ask appropriate questions - Provide accurate information |
| Underpinning Knowledge and Attitudes | Demonstrates knowledge of: - Organization requirements for written and electronic communication methods - Effective verbal communication methods |
| Underpinning Skills | Demonstrates skills to: - Organize information - Understand and convey intended meaning - Participate in variety of workplace discussions - Comply with organization requirements for the use of written and electronic communication methods |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Lead Small Teams |
| Unit Code | IND SHA3 15 0616 |
| Unit Descriptor | This unit covers the skills, knowledge and attitudes required to determine individual and team development needs and facilitate the development of the work group. |

| Elements | Performance Criteria |
| --- | --- |
| - Provide team leadership | - Learning and development needs are systematically identified and implemented in line with organizational requirements. - Learning plan is collaboratively developed and implemented to meet individual and group training and developmental needs. - Individuals are encouraged to self-evaluate performance and areas identified for improvement. - Feedback on performance of team members is collected from relevant sources and compared with established team learning process. |
| - Foster individual and organizational growth | - Learning and development program goals and objectives are identified to match the specific knowledge and skills requirements of competence standards. - Learning delivery methods are made appropriate to the learning goals, the learning style of participants and availability of equipment and resources. - Workplace learning opportunities and coaching/ mentoring assistance are provided to facilitate individual and team achievement of competencies. - Resources and timelines required for learning activities are identified and approved in accordance with organizational requirements. |
| - Monitor and evaluate workplace learning | - Feedback from individuals or teams is used to identify and implement improvements in future learning arrangements. - Outcomes and performance of individuals/teams are assessed and recorded to determine the effectiveness of development programs and the extent of additional support. - Modifications to learning plans are negotiated to improve the efficiency and effectiveness of learning. - Records and reports of competence are maintained within organizational requirement. |
| - Develop team commitment and cooperation | - Open communication processes are used by team to obtain and share information. - Decisions are reached by the team in accordance with its agreed roles and responsibilities. - Mutual concern and camaraderie are developed in the team. |
| - Facilitate accomplishment of organizational goals | - Team members are made actively participatory in team activities and communication processes. - Individual and joint responsibility has been developed teams members for their actions. - Collaborative efforts are sustained to attain organizational goals. |

| Variable | Range |
| --- | --- |
| Learning and development needs | May include but not limited to: - Coaching, mentoring and/or supervision - Formal/informal learning program - Internal/external training provision - Work experience/exchange/opportunities - Personal study - Career planning/development - Performance appraisals - Workplace skills assessment & Recognition of prior learning |
| Organizational requirements | May include but not limited to: - Quality assurance and/or procedures manuals - Goals, objectives, plans, systems and processes - Legal and organizational policy/guidelines and requirements - Safety policies, procedures and programs - Confidentiality and security requirements - Business and performance plans - Ethical standards - Quality and continuous improvement processes and standards |
| Feedback on performance | May include but not limited to: - Formal/informal performance appraisals - Obtaining feedback from supervisors and colleagues - Obtaining feedback from clients - Personal and reflective behavior strategies - Routine and organizational methods for monitoring service delivery |
| Learning delivery methods | May include but not limited to: - On the job coaching or mentoring - Problem solving - Presentation/demonstration - Formal course participation - Work experience and Involvement in professional networks - Conference/seminar attendance and induction |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Demonstrates skills and knowledge to: - identify and implement learning opportunities for others - give and receive feedback constructively - facilitate participation of individuals in the work of the team - negotiate learning plans to improve the effectiveness of learning - prepare learning plans to match skill needs - access and designate learning opportunities |
| Underpinning Knowledge and Attitude | Demonstrates knowledge of: - coaching and mentoring principles - how to work effectively with team members who have diverse work styles, aspirations, cultures and perspective - how to facilitate team development and improvement - methods and techniques for eliciting and interpreting feedback - methods for identifying and prioritizing personal development opportunities and options - career paths and competence standards in the industry |
| Underpinning Skills | Demonstrates skills to: - read and understand a variety of texts, prepare general information and documents according to target audience; spell with accuracy; use grammar and punctuation effective relationships and conflict management - receive feedback and report, maintain effective relationships and conflict management - organize required resources and equipment to meet learning needs - provide support to colleagues - organize information; assess information for relevance and accuracy; identify and elaborate on learning outcomes - facilitation skills to conduct small group training sessions - relate to people from a range of social, cultural, physical and mental backgrounds |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written exam - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the workplace or in a simulated workplace setting |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Improve Business Practice |
| Unit Code | IND SHA3 16 0616 |
| Unit Descriptor | This unit covers the knowledge, skills and attitudes required in promoting, improving and growing business operations. |

| Elements | Performance Criteria |
| --- | --- |
| - Diagnose the business | - Sources data is identified; data required for diagnosis is determined and acquired based on the business diagnosis toolkit. - Value chain analysis is conducted. - SWOT analysis of the data is undertaken. - Competitive advantage of the business is determined from the data. |
| - Benchmark the business | - Product or service to be benchmarked is identified and selected. - Sources of relevant benchmarking data are identified. - Key indicators are selected for benchmarking in consultation with key stakeholders. - Key indicators of own practice are compared with benchmark indicators. - Areas of improvements are identified. |
| - Develop plans to improve business performance | - A consolidated list of required improvements is developed. - Cost-benefit analysis is determined for required improvements. - Work flow changes resulting from proposed improvements are determined. - Proposed improvements are ranked according to agreed criteria. - An action plan is developed and agreed to implement the top ranked improvements. - Organizational structures are checked to ensure they are suitable. |
| - Develop marketing plans | - The practice vision statement is reviewed. - Practice objectives are developed/ reviewed. - Market research is conducted and result is obtained. - Target markets are identified/ refined. - Market position is developed/ reviewed. - Practice brand is developed. - Benefits of products or services are identified. - Promotion tools are selected and developed. |
| - Develop business growth plans | - Plans are developed to increase profitability - Proposed plans are ranked according to agreed criteria. - An action plan is developed and agreed to implement the top ranked plans. - Business work practices are reviewed to ensure they support growth plans. |
| - Implement and monitor plans | - Implementation plan is developed in consultation with all relevant stakeholders. - Success indicators of the plan are agreed. - Implementation is monitored against agreed indicators. - Implementation is adjusted as required. |

| Variable | Range |
| --- | --- |
| Data sources | May include but not limited to: - Primary data and secondary sources |
| Data required | May include but not limited to: - Organization capability - Appropriate business structure - Level of client service which can be provided - Internal policies, procedures and practices - Staff levels, capabilities and structure - Market and market definition - Market changes/market segmentation - Market consolidation/fragmentation - Revenue - Level of commercial activity - Expected revenue levels, short and long term - Revenue growth rate - Break even data - Pricing policy - Revenue assumptions - Business environment - Economic conditions - Social factors - Demographic factors - Technological impacts - Political/legislative/regulative impacts - Competitors, competitor pricing and response to pricing - Competitor marketing/branding - Competitor products |
| SWOT analysis | May include but not limited to: - Internal strengths such as staff capability, recognized quality - Internal weaknesses such as poor morale, under-capitalization, poor technology - External opportunities such as changing market and economic conditions - External threats such as industry fee structures, strategic alliances, competitor marketing |
| Competitive advantage | May include but not limited to: - Quality - Pricing - Cost - Location - Technology - Delivery - Timeframe - Promotion - Niche marketing - Support from government |
| Key indicators | May include but not limited to: - Staffing - Cost and expenses - Personnel productivity (particularly of principals) - Goodwill - Profitability - Price structure - Customers base - Productivity - Quality - System |
| Organizational structures | May include but not limited to: - Lines of authority and reporting relationship |
| Objectives | May include but not limited to: - Market share growth - Revenue growth - Profitability - Productivity - Innovation |
| Market position | May include but not limited to: - The goods or service provided - Product mix - The core product - what is bought - The tangible product - what is perceived - The augmented product - total package of consumer - Features/benefits - Product differentiation from competitive products - New/changed products - Price and pricing strategies (cost plus, supply/demand, ability to pay, etc.) - Pricing objectives (profit, market penetration, etc.) - Cost components - Market position - Distribution strategies - Marketing channels - Promotion - Target audience - Communication |
| Practice brand | May include but not limited to: - Practice image - Practice logo/letterhead/signage - Phone answering protocol - Facility decor - Slogans - Templates for communication/invoicing - Style guide - Writing style - AIDA (Attention, Interest, Desire and Action) |
| Benefits | May include but not limited to: - Features as perceived by the client - Benefits as perceived by the client |
| Promotion tools | May include but not limited to: - Networking and referrals - Seminars - Sales promotion - Advertising - Personal selling - Press releases - Publicity and sponsorship - Brochures - Newsletters (print and/or electronic) - Websites - Direct mail - Telemarketing/cold calling |
| Ranking | May include but not limited to: - Importance - Urgency - Technology - Resource availability |
| Relevant stockholders | May include but not limited to: - Micro and Small Enterprises development - Non-Government Organizations (NGOs) - Finance institutions - Capital goods leasing enterprise |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Demonstrates skills and knowledge of: - Identifying the key indicators of business performance - Identifying the key market data for the business - A wide range of available information sources - Acquiring information not readily available within a business - Analyzing data and determine areas of improvement - Negotiating required improvements to ensure implementation - Evaluating systems against practice requirements - Forming recommendations and/or make recommendations - Assessing the accuracy and relevance of information |
| Underpinning Knowledge and Attitudes | Demonstrates knowledge of: - Data gathering and analysis - Value chain analysis - SWOT analysis - Competitive advantage - Cost benefit analysis - Target market - Marketing principles - Organizational structure - Marketing mix - Promotion mix - Market position - Branding ProfitabilityDemonstrates knowledge of: - Data gathering and analysis - Value chain analysis - SWOT analysis - Competitive advantage - Cost benefit analysis - Target market - Marketing principles - Organizational structure - Marketing mix - Promotion mix - Market position - Branding - Profitability |
| Underpinning Skills | Demonstrates skill in: - Benchmarking skills - Communication skills - Computers kills to manipulate data and present information - Negotiation skills - Preparing action plan - Conducting market research - Identifying target market - Identifying suitable marketing mix - Preparing promotional tools - Problem solving - Planning skills - Monitoring and evaluation - Ability to acquire and interpret relevant data - Use of market intelligence - Development and implementation strategies of promotion and growth plans - Ability to acquire and interpret required data, current practice systems and structures and sources of relevant benchmarking data - Applying methods of selecting relevant key benchmarking indicators - Communication skills - Working and consulting with others when developing plans for the business - Negotiation skills - Using computers to manipulate, present and distribute information |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

| Occupational Standard: Small and Heavy Armaments Servicing and Maintenance Level III |  |
| --- | --- |
| Unit Title | Prevent and Eliminate MUDA |
| Unit Code | IND SHA3 17 0616 |
| Unit Descriptor | This unit of competence covers the knowledge, skills and attitude required by a worker to prevent and eliminate MUDA/wastes in his/her their workplace. It covers responsibility for the day-to-day operation of the work and ensures Kaizen elements are continuously improved and institutionalized. |

| Elements | Performance Criteria |
| --- | --- |
| - Prepare for work. | - Work instructions are used to determine job requirements, including method, material and equipment. - Job specifications are read and interpreted following working manual. - OHS requirements, including dust and fume collection, breathing apparatus and eye and ear personal protection needs are observed throughout the work. - Appropriate material is selected for work. - Safety equipment and tools are identified and checked for safe and effective operation. |
| - Identify MUDA. | - Plan of MUDA identification is prepared and implemented. - Causes and effects of MUDA are discussed. - Tools and techniques are used to draw and analyze current situation of the work place. - Wastes/MUDA are identified and measured based on relevant procedures. - Identified and measured wastes are reported to relevant personnel. |
| - Eliminate wastes/MUDA. | - Plan of MUDA elimination is prepared and implemented. - Necessary attitude and the ten basic principles for improvement are adopted to eliminate waste/MUDA. - Tools and techniques are used to eliminate wastes/MUDA based on the procedures and OHS. - Wastes/MUDA are reduced and eliminated in accordance with OHS and organizational requirements. - Improvements gained by elimination of waste/MUDA are reported to relevant bodies. |
| - Prevent occurrence of wastes/MUDA. | - Plan of MUDA prevention is prepared and implemented. - Standards required for machines, operations, defining normal and abnormal conditions, clerical procedures and procurement are discussed and prepared. - Occurrences of wastes/MUDA are prevented by using visual and auditory control methods. - Waste-free workplace is created using 5W and 1Hsheet. - The completion of required operation is done in accordance with standard procedures and practices. - The updating of standard procedures and practices is facilitated. - The capability of the work team that aligns with the requirements of the procedure is ensured. |

| Variable | Range |
| --- | --- |
| OHS requirements | May include but not limited to: - Are to be in accordance with legislation/ regulations/codes of practice and enterprise safety policies and procedures. This may include protective clothing and equipment, use of tooling and equipment, workplace environment and safety, handling of material, use of firefighting equipment, enterprise first aid, hazard control and hazardous materials and substances. - Personal protective equipment is to include that prescribed under legislation/regulations/codes of practice and workplace policies and practices. - Safe operating procedures are to include, but are not limited to the conduct of operational risk assessment and treatments associated with workplace organization. - Emergency procedures related to this unit are to include but may not be limited to emergency shutdown and stopping of equipment, extinguishing fires, enterprise first aid requirements and site evacuation. |
| Safety equipment and tools | May include but not limited to: - dust masks / goggles - glove - working cloth - first aid - safety shoes |
| Tools and techniques | May include but not limited to: - Plant Layout - Process flow - Other Analysis tools - Do time study by work element - Measure Travel distance - Take a photo of workplace - Measure Total steps - Make list of items/products, who produces them and who uses them & those in warehouses, storages etc. - Focal points to Check and find out existing problems - 5S - Layout improvement - Brainstorming - Andon - U-line - In-lining - Unification - Multi-process handling & Multi-skilled operators - A.B. control (Two point control) - Cell production line - TPM (Total Productive Maintenance) |
| Relevant procedures | May include but not limited to: - Make waste visible - Be conscious of the waste - Be accountable for the waste. - Measure the waste. |
| The ten basic principles for improvement | May include but not limited to: - Throw out all of your fixed ideas about how to do things. - Think of how the new method will work- not how it won. - Don’t accept excuses. Totally deny the status quo. - Don’t seek perfection. A 5o percent implementation rate is fine as long as it’s done on the spot. - Correct mistakes the moment they are found. - Don’t spend a lot of money on improvements. - Problems give you a chance to use your brain. - Ask “why?” At least five times until you find the ultimate cause. - Ten people’s ideas are better than one person’s. - Improvement knows no limits. |
| Visual and auditory control methods | May include but not limited to: - Red Tagging - Sign boards - Outlining - Andons - Kanban, etc. |
| 5W and 1H | May include but not limited to: - Who - What - Where - When - Why - How |

| Evidence Guide |  |
| --- | --- |
| Critical Aspects of Competence | Demonstrates skills and knowledge to: - discuss why wastes occur in the workplace - discuss causes and effects of wastes/MUDA in the workplace - analyze the current situation of the workplace by using appropriate tools and techniques - identify, measure, eliminate and prevent occurrence of wastes by using appropriate tools and techniques - use 5W and 1H sheet to prevent |
| Underpinning Knowledge and Attitudes | Demonstrates knowledge of: - Targets of customers and manufacturer/service provider - Traditional and kaizen thinking of price setting - Kaizen thinking in relation to targets of manufacturer/service provider and customer - value - The three categories of operations - the 3“MU” - waste/MUDA - wastes occur in the workplace - The 7 types of MUDA - The Benefits of identifying and eliminating waste - Causes and effects of 7 MUDA - Procedures to identify MUDA - Necessary attitude and the ten basic principles for improvement - Procedures to eliminate MUDA - Prevention of wastes - Methods of waste prevention - Definition and purpose of standardization - Standards required for machines, operations, defining normal and abnormal conditions, clerical procedures and procurement - Methods of visual and auditory control - TPM concept and its pillars. - Relevant OHS and environment requirements - Plan and report - Method of communication |
| Underpinning Skills | Demonstrates skills to: - draw & analyze current situation of the work place - use measurement apparatus (stop watch, tape, etc.) - calculate volume and area - use and follow checklists to identify, measure and eliminate wastes/MUDA - identify and measure wastes/MUDA in accordance with OHS and procedures - use tools and techniques to eliminate wastes/MUDA in accordance with OHS procedure - apply 5W and 1H sheet - update and use standard procedures for completion of required operation - work with others - read and interpret documents - observe situations - solve problems - communicate - gather evidence by using different means - report activities and results using report formats |
| Resources Implication | Access is required to real or appropriately simulated situations, including work areas, materials and equipment, and to information on workplace practices and OHS practices. |
| Methods of Assessment | Competence may be assessed through: - Interview / Written Test - Observation / Demonstration with Oral Questioning |
| Context of Assessment | Competence may be assessed in the work place or in a simulated work place setting. |

Acknowledgement

We wish to extend thanks and appreciation to the many representatives of business, industry, academe and government agencies who donated their time and expertise to the development of this occupational standard.

We would like also to express our appreciation to the Staff and Experts of Maj.Gen.Mulugeta Buli Technical College, Federal TVET Agency and Ministry of Education (MoE) who made the development of this occupational standard possible.

This occupational standard was developed in May 2013 at Maj.Gen.Mulugeta Buli Technical College.